GMS 10.9

GMS 10.9 Tutorial

MODFLOW-USG – Quadtree

Create a MODFLOW-USG model with a locally refined UGrid using GMS

Objectives

GMS supports building MODFLOW-USG models with quadtree UGrids. This tutorial shows how to generate a smoothed quadtree UGrid for a complex model.

Time

25–40 minutes

Required Components

GMS Core

MODFLOW Interface

Prerequisite Tutorials

MODFLOW – Conceptual Model Approach I

UGrid Creation

1 Introduction 2 2 Getting Started 3 2.1 Open the Starting Project 3 2.2 Save with a Different Name 4 3 Create the Conceptual Model 4 3.1 Create the Areal Coverage 4 3.2 Setup the Sources and Sinks Coverage 5 3.3 Assign Specified Head 5 3.4 Assign Rivers 6 3.5 Assign Wells 7 3.6 Refinement for Grid Generation 7 4 Create the Unstructured Grid 8 5 Create a MODFLOW-USG Model 10 6 Set Grid Elevations 10 7 Map to MODFLOW 11 8 Save and Run MODFLOW 12 9 Conclusion 13

# Introduction

MODFLOW-USG was developed to allow for a wide variety of grid geometries, including the ability for increased grid resolution around rivers, wells, and other areas of interest. One such type of unstructured grid (UGrid) is a smoothed quadtree UGrid. In a smoothed quadtree UGrid, the largest sized cell is refined near areas of interest by splitting each successive neighboring cell into four cells until the desired cell size is reached.

GMS has the ability to easily generate smoothed quadtree UGrids from a conceptual model. Areas of interest can be identified on specific conceptual model objects including points, arcs, and polygons.

The problem in this tutorial is illustrated in Figure 1. The model is based on the quadtree example problem included with MODFLOW-USG. The model consists of a specified head boundary along the ocean to the right of the model. It also includes a network of canals modeled using the RIV package and wells modeled using the WEL package.

The locations of the boundary conditions are provided by shapefiles and the elevations by raster files. The generated grid is one layer thick and contains base cells (the largest cells) that are 800 m by 800 m. At specified head and river boundary conditions, the cells are refined down to 100 m; and at wells, the cells are refined down to 50 m.

This tutorial will demonstrate and discuss:

- Importing an existing GMS project containing model shapefiles.

- Creating a conceptual model using the shapefile geometry.

- Turning on refinement for grid generation.

- Creating a quadtree UGrid.

- Creating a MODFLOW-USG model.

- Mapping to MODFLOW.

- Running the model and examining the results.

Figure 1      The Biscayne Bay area of Miami, Florida

# Getting Started

Do the following to get started:

- If necessary, launch GMS.

- If GMS is already running, select File | New to ensure that the program settings are restored to their default state.

## Open the Starting Project

To open a GMS project that already has the shapefiles which contain the model geometry:

- Click Open  to bring up the Open dialog.

- Select “Project Files (*.gpr)” from the Files of type drop-down.

- Browse to the Quadtree\Quadtree\ directory and select “start.gpr”.

- Click Open to import the project and exit the Open dialog.

## Save with a Different Name

Before making any changes, it is best to save the project under a new name.

- Select File | Save As… to bring up the Save As dialog.

- Select “Project Files (*.gpr)” from the Save as type drop-down.

- Enter “biscayne.gpr” as the File name.

- Click Save to save the project under the new name and close the Save As dialog.

Save  periodically throughout the project.

# Create the Conceptual Model

To create a conceptual model:

- Right-click in a blank space in the Project Explorer and select New | Conceptual Model… to open the Conceptual Model Properties dialog.

- Enter “Biscayne” as the Name.

- Click OK to close the Conceptual Model Properties dialog.

## Create the Areal Coverage

To create a coverage with horizontal conductivity and recharge areal properties:

- Right-click on “ Biscayne” in the Project Explorer and select New Coverage… to open the Coverage Setup dialog.

- Enter “Areal” as the Coverage name.

- In the Areal Properties* column, turn on Recharge rate and Horizontal K.

- Turn on Use to define model boundary (active area).

- Click OK to create the coverage and close the Coverage Setup dialog.

Next, to convert the shape files to feature objects and build polygons:

- Under “ GIS Layers” in the Project Explorer, turn on “ Biscayne.shp”.

- Right-click on “ Biscayne.shp” and select Convert To | Feature Objects to open the GIS to Feature Objects Wizard dialog.

- Click Next > to go to the Step 2 of 3 page of the GIS to Feature Objects Wizard dialog.

- Click Next > to go to the Step 3 of 3 page of the GIS to Feature Objects Wizard dialog.

- Click Finish to create the feature objects and close the GIS to Feature Objects Wizard dialog.

- Select “ Areal” in the Project Explorer to make it active.

- Click the Build Polygons  macro.

- Right-click on “ Areal” and select Attribute Table… to open the Attribute Table dialog.

- From the Feature type drop-down, select “Polygons”.

- In the All row in the Recharge rate (m/d) column, enter “1.0e-4”.

- In the All row in the Horizontal K (m/d) column, enter “400.0”.

- Click OK to save the values and close the Attribute Table dialog.

## Setup the Sources and Sinks Coverage

To create a coverage containing the specified head, river, and well boundary conditions:

- Right-click on “ Biscayne” in the Project Explorer and select New Coverage… to bring up the Coverage Setup dialog.

- Enter “Sources-Sinks” as the Coverage name.

- In the Sources/Sinks/BCs column, turn on Wells, Specified Head (IBOUND), and River.

- Click OK to create the coverage and close the Coverage Setup dialog.

Now it is possible to copy the specified head, river, and well geometry from the shapefile to the coverage.

- Turn on “ Wells.shp” in the Project Explorer.

- Right-click on “ Wells.shp” and select Convert To | Feature Objects to bring up the GIS to Feature Objects Wizard dialog.

- Make certain both “Biscayne.shp” and “Wells.shp” are selected under Select shapefiles to map.

- Click Next > to go to the Step 2 of 4 page of the GIS to Feature Objects Wizard dialog.

- Click Next > to go to the Step 3 of 4 page of the GIS to Feature Objects Wizard dialog.

- Click Next > to go to the Step 4 of 4 page of the GIS to Feature Objects Wizard dialog.

- Click Finish to create the feature objects and close the GIS to Feature Objects Wizard dialog.

- Turn off “ Biscayne.shp” and “ Wells.shp” in the Project Explorer.

## Assign Specified Head

Now it is necessary to set the arc types by setting up the specified head arcs.

- Select “ Sources-Sinks” in the Project Explorer to make it active.

- Using the Select Arcs  tool, select the 14 arcs along the coastline by holding down the Shift key and clicking on each arc. Zoom  in if necessary.

- Right-click on one of the selected arcs and choose Attribute Table… to open the Attribute Table dialog.

- Select “spec. head (IBOUND)” from the drop-down in the All row of the Type column. This changes the arcs to specified head.

- Click OK to save the values and close the Attribute Table dialog.

- Do not unselect the arcs.

The map should now look similar to Figure 2 (note that the arcs are all still selected). If necessary, select any missed arcs and assign the boundary condition by following the steps above. Once done, reselect all of the coastline arcs by following step 2 above.

Figure 2      Specified head boundary condition applied to arcs on coastline

## Assign Rivers

To set up the river arcs:

- With the coastal arcs still selected, select Edit | Invert Selection.

- Right-click on “ Sources-Sinks” and select Attribute Table… to bring up the Attribute Table dialog.

- In the All row, select “river” from the drop-down in the Type column.

- In the All row, enter “5000.0” in the Cond. (m^2/d)/(m) column.

- Select “Nodes” from the Feature type drop-down.

- Select “river” from the BC type drop-down.

To set the river stage and bottom elevation, the rasters that were included in the starting project will be used.

- In the All row, click on the  button in the Head-Stage (m) column and select “<Raster>” from the drop-down to bring up the Select Raster dialog.

- Select “RiverStage.tif” and click OK to close the Select Raster dialog.

- In the All row, click on the  button in the Bot. elev. (m) column and select “<Raster>” from the drop-down to bring up the Select Raster dialog.

- Select “RiverBottoms.tif” and click OK to close the Select Raster dialog.

- Click OK to close the Attribute Table dialog.

- Do not unselect the arcs.

## Assign Wells

To assign the wells:

- Right-click on “ Sources-Sinks” and select Attribute Table… to bring up the Attribute Table dialog.

- Select “Points” from the Feature type drop-down.

- In the All row, select “well” from the drop-down in the Type column.

- In the All row, enter “-700.0” in the Flow rate (m^3/d) column.

- Click OK to save the values and close the Attribute Table dialog.

- Deselect the arcs by clicking in a blank area in the Graphics Window.

The map should now appear similar to Figure 3.

Figure 3      Biscayne area conceptual model

## Refinement for Grid Generation

To generate the quadtree grid, it is necessary to turn on refinement for the areas of interest. For this model, this involves turning on refinement for the specified head, river boundary conditions, and wells.

- Right-click on “ Sources-Sinks” and select Coverage Setup… to open the Coverage Setup dialog.

- In the Sources/Sinks/BCs column, turn on Refinement.

- Click OK to close the Coverage Setup dialog.

Now it is necessary to turn on refinement for each object, beginning with the wells.

- Right-click on “ Sources-Sinks” and select Attribute Table… to open the Attribute Table dialog.

- Select “Points” from the Feature type drop-down.

- In the All row, check the box in the Refine column.

- In the All row, enter “60.0” in the Base size (m) column. Scroll over to the column if necessary.

When the grid is generated, the cells that contain the wells will be sized to the first cell size below “60.0”. To generate the grid, the cell size starts at the base cell size and then gets divided by two until the size is below “60.0”. The base cell is 800 m, so a Base size of 60 m will result in cell refined to 400 m, 200 m, 100 m, and then 50 m.

The next step is to set the refinement for the specified head and river arcs. For the river arcs, it is ideal for the cells to be refined to 100 m, so set the size slightly above 100 m.

- Select “Arcs” from the Feature type drop-down.

- In the All row, check the box in the Refine column.

- In the All row, enter “110.0” in the Base size (m) column.

- Click OK to close the Attribute Table dialog.

# Create the Unstructured Grid

Now it is necessary to set the dimensions of the unstructured grid by using a grid frame. The quadtree UGrid can be generated without a grid frame, but a grid frame gives more precise control over the size of the generated cells.

For this model, it is ideal to set the base grid cell size (or largest cell size) to 800 m. To do so, use a grid frame set with dimensions that are a multiple of 800 m.

- Right-click in a blank space in the Project Explorer and select New | Grid Frame. A new grid frame should appear.

- Right-click on “ Grid Frame” in the Project Explorer and select Fit to Active Coverage.

- Right-click on “ Grid Frame” and select Properties… to open the Grid Frame Properties dialog.

- In the Value column, enter the values from the table below:

| Origin x: | 543750.0 |
| --- | --- |
| Origin y: | 2793450.0 |
| Origin z: | 0.0 |
| Dimension x: | 44000.0 |
| Dimension y: | 79200.0 |
| Dimension z: | 10.0 |

- Click OK to exit the Grid Frame Properties dialog.

The grid frame will adjust to the more specific dimensions just entered. This gives a base grid with 44 columns and 99 rows.

Now it is possible to create the quadtree UGrid. While a grid frame can be used to determine the dimensions of the grid cells, the polygons in the active coverage determine the boundary of the generated cells. If there are no polygons in the active coverage, the generated cells will fill the grid frame.

For the grid, use the polygons in the “Areal” coverage.

- Select “ Areal” in the Project Explorer to make it active.

- Right-click on “ Grid Frame” and select Map To | UGrid to open the Map → UGrid dialog.

- From the UGrid type drop-down, select “Quadtree / Octree”.

- From the Arc/poly refinement drop-down, select “Quadtree on assigned layers”.

- In the X-Dimension section, select Cell size as the Cell size method and enter “800.0” as the Cell size.

- In the Y-Dimension section, select Cell size as the Cell size method and enter “800.0” as the Cell size.

- In the Z-Dimension section, enter “1” as the Number of cells.

- The options in the Map → UGrid dialog should match those shown in Figure 4.

Figure 4      Map → UGrid dialog

- Click OK to exit the Map → UGrid dialog and generate the unstructured grid.

- Right-click on “ Biscayne.tif” in the Project Explorer and select Transparency… to bring up the Layer Transparency dialog.

- Set the Transparency slider to “30%” and click OK to close the Layer Transparency dialog. This makes the UGrid easier to see.

The new quadtree UGrid should appear similar to Figure 5.

Figure 5      Quadtree grid

It is now possible to check how many cells were generated.

- Right-click on “ ugrid” in the Project Explorer and select Properties… to open the UGrid Properties dialog.

The generated unstructured grid has around 28,000 cells. Generating a structured grid for the same model using a cell size of 100 m results in a grid with about 185,000 cells.

- Click OK to exit the UGrid Properties dialog.

# Create a MODFLOW-USG Model

Now it is possible to create the MODFLOW-USG model.

- Select MODFLOW | New Simulation… to bring up the MODFLOW Global/Basic Package dialog.

- Click OK to accept the defaults and exit the MODFLOW Global/Basic Package dialog.

# Set Grid Elevations

The top and bottom elevations for the model need to be set up. This tutorial will interpolate raster elevations to MODFLOW layers.

- Right-click on “ Tops.tif” in the “ GIS Layers” folder in the Project Explorer and select Interpolate To | MODFLOW Layers… to bring up the Interpolate to MODFLOW Layers dialog.

The “Tops.tif” dataset has automatically been mapped to “Top Elevations Layer 1” in the array in the Dataset → MODFLOW data section. Now map “Bottoms.tif” to “Bottom Elevation Layer 1”.

- In the Dataset → MODFLOW data section, select “Tops.tif” under the Dataset column and select Unmap.

- In the Rasters list, select “Bottoms.tif”.

- In the MODFLOW data list, click on “Bottom Elevations Layer 1”.

- Click Map to add the mapping to the array in the Dataset → MODFLOW data section.

- Click OK to interpolate the top and bottom values to the MODFLOW model and close the Interpolate to MODFLOW Layers dialog.

# Map to MODFLOW

To assign the conceptual model values to the MODFLOW model:

- Select “ Biscayne” in the Project Explorer to make it active.

- Select Feature Objects | Map → MODFLOW to bring up the Map → Model dialog.

- Click OK to accept the defaults and close the Map → Model dialog. It may take a moment for the model to load.

The boundary condition symbols for specified head, rivers, and wells should appear (Figure 6).

Figure 6      MODFLOW boundary conditions assigned from the conceptual model

# Save and Run MODFLOW

Now it is possible to run MODFLOW.

- Save  the project.

- Click the Run MODFLOW  macro to bring up the MODFLOW model wrapper dialog.

- When the model finishes, turn on Read solution on exit and Turn on contours (if not on already).

- Click Close to import the solution and exit the MODFLOW model wrapper dialog.

The Graphics Window should appear similar to Figure 7.

Figure 7      MODFLOW-USG head contours

# Conclusion

This concludes the “MODFLOW-USG – Quadtree” tutorial. The following key concepts were discussed and demonstrated in this tutorial:

- GMS supports generating smoothed quadtree UGrids that can be used with MODFLOW-USG.

- The localized cell refinement for a generated grid can be controlled by turning on refinement and setting the base size in the conceptual model.

- Polygons in the active coverage determine the area in which cells are generated.

- A grid frame can be used to further control the size and position of the generated cells.

Panday, Sorab; Langevin, Christian D.; Niswonger, Richard G.; Ibaraki, Motomu; and Hughes, Joseph D., (2013). “MODFLOW–USG version 1: An Unstructured Grid Version of MODFLOW for Simulating Groundwater Flow and Tightly Coupled Processes Using a Control Volume Finite-Difference Formulation” in Techniques and Methods 6–A45, U.S. Geological Survey, 66 p.

Panday, et al (2013).