GMS 10.9

GMS 10.9 Tutorial

Online Maps

Using free, dynamic, online map data in GMS

Objectives

Learn how free online map data can quickly and easily be used to dynamically update background maps and aerial photography in GMS.

Time

10–20 minutes

Required Components

An internet connection

Prerequisite Tutorials

Getting Started

z

1 Introduction 2 1.1 Getting Started 2 2 Adding Online Maps 2 3 Appearance 4 3.1 Changing the Display Order 4 3.2 Transparency 4 4 Exporting an Image File 4 5 Image Properties and Save Location 5 6 Map Locator 5 7 Other Online Maps 6 8 Conclusion 6

# Introduction

Online maps are free maps, aerial photos, and DEMs that can be downloaded from the internet and displayed in the background in plan view. When zooming in and out or panning the view, online maps download the required image data from the internet and update automatically. Depending on the speed of the internet connection, this usually takes just a few moments. Online maps can be saved locally as image files. Online maps can be helpful in the initial stages of setting up a groundwater model.

In this tutorial, the following topics will be covered:

- Add an online map showing an aerial photo of the area around Moab, Utah.

- Zoom and pan to see how the image updates automatically.

- Add a second online map and explore image order and transparency.

- Create a local image file from the online map.

- Use the Map Locator  to reposition the display.

## Getting Started

To get started, do the following:

- If necessary, launch GMS.

- If GMS is already running, select File | New… to ensure that the program settings are restored to their default state.

# Adding Online Maps

To start, add an online map. Before adding the map, notice the current projection displayed in the bottom right of the Graphics Window. It should say “No projection, Feet (U.S. Survey).” This means that no global projection has been defined.

- Click the Add Online Maps  macro to bring up the Map Locator dialog.

- In the Search field, enter “Moab, Utah” and click Enter.

- Click OK to close the Map Locator dialog.

A warning will appear stating that a new global projection needs to be set. Had a matching projection already been set, then this warning would not have appeared.

- Click OK to close the warning dialog and open the Display Projection dialog.

By default, the global projection of the image location selected in the Map Locator  is populated in the Display Projection dialog.

- Click OK to close the Display Projection dialog and open the Get Online Maps dialog.

- Select World Imagery and click OK to exit the Get Online Maps dialog.

After a few moments (depending on speed of the internet connection), an aerial photo of the city of Moab, Utah will appear in the Graphics Window. Notice the projection displayed in the bottom right of the Graphics Window now says “UTM” and “NAD83...” Because the project had no data, GMS switched the display projection to UTM.

- Using the scroll wheel on the mouse or the Zoom  tool, zoom out repeatedly. Notice that the image updates with different resolution images as the zoom level changes.

- Use the Pan  tool to pan around the area. Notice how the image updates.

Notice the icon for the online map item in the Project Explorer looks like the typical image item icon but with the addition of a globe in the bottom right corner , indicating it is an online map. As the image is downloading from the internet, the text next to the item in the Project Explorer will change to “(processing) World Imagery”. Once the download is complete, the “(processing)” is removed.

Figure 1      Online satellite map of Moab, Utah

The next step is to add a second online map.

- Click the Add Online Map  macro to bring up the Get Online Maps dialog.

The Map Locator  dialog does not open this time because data already exists in the project.

- Select World Street Map and click OK to close the Get Online Maps dialog.

After a few moments, a street map of the area will appear in the Graphics Window (Figure 2).

Figure 2      Online street map of Moab, Utah

# Appearance

GMS provides tools for how online maps are displayed in the Graphics Window.

## Changing the Display Order

Images are displayed top to bottom, in the order they appear in the Project Explorer. To change the order, do the following:

- In the Project Explorer, select and drag the “ World Street Map” item so that it is underneath the “ World Imagery” item.

Notice that the aerial photo is visible once again.

- Place the “ World Street Map” back on top by dragging it so it is above “ World Imagery”.

## Transparency

The transparency of online maps can be set just as with other images.

- Right-click on the “ World Street Map” item in the Project Explorer and select Transparency… to bring up the Layer Transparency dialog.

- Use the slider to change the Transparency to around “40%”.

- Click OK to close the Layer Transparency dialog.

The “ World Imagery” will now be visible through the “ World Street Map” image.

# Exporting an Image File

A copy of the online map currently displayed in the Graphics Window can be saved as an image file. The image file is saved locally so it is accessible without an internet connection.

- In the Project Explorer, right-click on “ World Street Map” and select Export… to bring up the Resample and Export Raster dialog.

It is often desirable to save the image at a higher resolution than is currently displayed on screen. Because the exported image is limited to the resolution at which it was saved, it will not update with higher resolution online imagery when zooming in.

- Enter “2.0” in the Resampling ratio field.

- Turn on Add to project after saving.

- Click OK to close the Resample and Export Raster dialog and bring up the Save As dialog.

- Enter “MoabStreetmap.tif” in the File name field.

- Select “GeoTIFF Files (*.tif)” from the Save as type drop-down.

- Click Save to export the file and close the Save As dialog.

Once the download and export is complete, a third image titled “ MoabStreetmap.tif” will appear in the Project Explorer.

# Image Properties and Save Location

To view the folder where the exported TIFF is located, do the following:

- Right-click on the newly created “ MoabStreetmap.tif” and select Open Containing Folder to bring up the directory where the image file was saved.

The folder containing the exported image file will open in Windows Explorer.

To review the properties of the image that was just created, return to GMS and do the following:

- Right-click on “ MoabStreetmap.tif” and select Projection | Projection… to bring up the Projection – Project\GIS Layers\MoabStreetmap.tif dialog.

This dialog displays the projection for the image that was saved with the image when it was exported.

- Click Cancel to close the Projection– Project\GIS Layers\MoabStreetmap.tif dialog.

# Map Locator

When the first online map was downloaded, GMS opened the Map Locator dialog so that the desired map location could be entered because there was no project data. The Map Locator dialog can be used at any time to change the Graphics Window view to show different places on the globe.

- Click the Map Locator  macro to bring up the Map Locator dialog. The current location in Moab will be shown.

- Use the mouse scroll wheel or the Zoom Out  macro to zoom out as far as possible.

A map of the entire world should now be visible. From this point, it is possible to Zoom In  on any location in the world.

- Zoom in  on a new location.

- When done, click OK to close the Virtual Earth Map Locator dialog.

A warning dialog will appear stating the new location is outside of the current projection.

- Click OK to close the warning and open the Display Projection dialog.

- Accept the new projection and click OK to close the Display Projection dialog.

Notice the “ World Street Map” and “ World Imagery” will update to show the new location.

# Other Online Maps

At this point, the user may want to experiment with the other online map types available in the Get Online Maps dialog.

# Conclusion

This concludes the “Online Maps” tutorial. The following key concepts were discussed and demonstrated:

- Importing and using different free online maps.

- Changing the appearance of the imported maps.

- Saving an online map to be used offline.

- Using the Map Locator  to view any location in the world.