GMS 10.9

GMS 10.9 Tutorial

SEAWAT – Goswami/Clement Experiment

Simulate transient movement of a saltwater wedge using SEAWAT

Objectives

This tutorial shows how to simulate a transient saltwater intrusion dataset as presented in the Goswami/Clement experiment.

.

Time

30–45 minutes

Required Components

GMS Core

MODFLOW Interface

MT3D

SEAWAT

Prerequisite Tutorials

MODFLOW – Grid Approach

MT3DMS – Grid Approach

1 Introduction 2 2 Getting Started 3 2.1 Importing the MODFLOW Project 3 2.2 Saving the Model with a New Name 4 3 Setting Up MT3DMS 4 3.1 Basic Transport Package 4 3.2 MT3DMS, Species, and Porosity 5 3.3 Dispersion Package 5 4 First SEAWAT Scenario (SS-1) 6 4.1 Initializing SEAWAT 6 4.2 Modifying the VDF Package 7 4.3 Saving and Running SEAWAT 7 4.4 Viewing the SS-1 Solution 7 5 Second SEAWAT Scenario (SS-2) 9 5.1 Saving the Model with a New Name 9 5.2 Initial Conditions for SS-2 9 5.3 Saving and Running SEAWAT 10 5.4 Viewing the SS-2 Solution 10 6 Third SEAWAT Scenario (SS-3) 12 6.1 Saving the Model with a New Name 12 6.2 Initial Conditions for SS-3 12 6.3 Saving and Running SEAWAT 13 6.4 Viewing the SS-3 Solution 13 7 Conclusion 16 8 Further Reading 16

# Introduction

This tutorial describes how to simulate the transient movement of a saltwater wedge in and out of an aquifer. This example problem is very similar to the experimental problem described in the 2007 Goswami and Clement article. The experimental setup used for this problem is shown in Figure 1. Although this is an unconfined flow problem, the groundwater gradients used are small and hence the problem can be simplified and modeled as a confined flow saltwater intrusion problem.

The experimental system has a constant saltwater boundary in the left and constant freshwater boundary in the right. The salt concentration used was 37 kg/m3 (or 0.037 g/cm3) and the density was 1026 kg/m3 (or 1.026 g/cm3). This tutorial demonstrates the steps used to define MODFLOW, MT3DMS, and SEAWAT input data creating this simulation.

Figure 1      Experimental setup used

# Getting Started

Do the following to get started:

- If necessary, launch GMS.

- If GMS is already running, select File | New to ensure that the program settings are restored to their default state.

## Importing the MODFLOW Project

A MODFLOW simulation with the correct units, a 3D grid, the correct LPF options, and cell properties has been created for this tutorial. The grid is 53.5 by 2.7 by 25.5 cm (xyz dimensions). MODFLOW has been run to generate the vertical contour lines.

Import it by doing the following:

- Click Open  to bring up the Open dialog.

- Select “Project Files (*.gpr)” from the Files of type drop-down.

- Browse to the saltwater directory and select “start.gpr”.

- Click Open to import the project and exit the Open dialog.

The project should appear similar to Figure 2.

Figure 2      The imported MODFLOW project

## Saving the Model with a New Name

Before proceeding with the tutorial, save the project under a new name:

- Select File | Save As… to bring up the Save As dialog.

- Select “Project Files (*.gpr)” from the Save as type drop-down.

- Enter “GC1.gpr” as the File name.

- Click Save to save the project under the new name and close the Save As dialog.

# Setting Up MT3DMS

## Basic Transport Package

To start this process, first create a new MT3DMS simulation

- Select “ grid”, then right-click on it and select New MT3DMS… to bring up the Basic Transport Package dialog.

- Click Stress Periods… to bring up the Stress Periods dialog.

- On row 1, for Length, enter “80.0”, and for Num Time Steps, enter “8”.

- Click OK to close the Stress Periods dialog.

- Click Output Control… to bring up the Output Control dialog.

- Below the Retardation factor section, select Print or save at specified times.

- Click Times… to bring up the Variable Time Steps dialog.

- Click Initialize Values… to bring up the Initialize Time Steps dialog.

- For Initial time step size, enter “1.0”.

- For Bias, enter “4.0”.

- For Maximum time step size, enter “5.0”.

- For Maximum simulation time, enter “80.0”.

- Click OK to close the Initialize Time Steps dialog.

Notice that the spreadsheet in the Variable Time Steps dialog has been populated with time steps ranging from “1.0” up to “80.0”.

- Click OK to close the Variable Time Steps dialog.

- Click OK to close the Output Control dialog.

## MT3DMS, Species, and Porosity

Do the following to set up the MT3DMS options:

- Click Packages… to bring up the MT3DMS/RT3D Packages dialog.

- Turn on Advection package, Dispersion package, and Source/sink mixing package then click OK to close the MT3DMS/RT3D Packages dialog.

- Click Define Species… to bring up the Define Species dialog.

- Click New to add a new species to the spreadsheet.

- In the Name column, enter “Salt” then click OK to close the Define Species dialog.

- Click Porosity… to bring up the Porosity dialog.

- Click Constant → Grid… to bring up the Grid Value dialog.

- For Constant value for grid, enter “0.385”.

- Click OK to close the Grid Value dialog.

- Click OK to close the Porosity dialog.

- Click OK to close the Basic Transport Package dialog.

## Dispersion Package

- Select MT3DMS | Dispersion Package… to bring up the Dispersion Package dialog.

- Click Longitudinal Dispersivity… to bring up the Longitudinal Dispersivity dialog.

- Click Constant → Grid… to bring up the Grid Value dialog.

- For Constant value for grid, enter “0.1” then click OK to close the Grid Value dialog.

- Click OK to close the Longitudinal Dispersivity dialog.

- Click OK to close the Dispersion Package dialog.

- Using the Select j  tool, click the top left cell to select all boundary cells in the leftmost column.

- Right-click and select Sources/Sinks… to bring up the MODFLOW/MT3DMS Sources/Sinks dialog.

- From the list on the left, select “MT3DMS: Point SS”.

- Click Add BC to create a new set of entries in the spreadsheet on the right.

- In the All row of the spreadsheet, under Type (ITYPE), use the drop-down to select “constant concentration”.

- In the All row, for Salt, enter “0.037”.

- Click OK to close the MODFLOW/MT3DMS Sources/Sinks dialog.

- Select MT3DMS | Check Simulation… to bring up the Model Checker dialog.

- Click Run Check.

There should be no errors or warnings.

- Click Done to close the Model Checker dialog.

# First SEAWAT Scenario (SS-1)

## Initializing SEAWAT

The objective of the Goswami/Clement experiment was to generate three steady-state profiles and two sets of transient simulations described by the boundary conditions shown in Figure 3. The saltwater head is fixed at 25.5 in all of the simulations. The freshwater head varies slightly in each simulation.

SEAWAT simulations use a combination of MODFLOW and MT3DMS packages along with VDF, a SEAWAT-specific package.

- Right-click on “ grid” and select New SEAWAT… to bring up the Global Options dialog.

- Turn on Include transport in simulation.

- In the Packages section, turn on Variable-Density Flow (VDF).

- Click OK to exit the Global Options dialog.

Figure 3      Boundary conditions simulated in the experimental study

## Modifying the VDF Package

For the first scenario, simulate the effect of salinity on the fluid density.

- Select SEAWAT | VDF Package… to open the SEAWAT VDF Package dialog.

- For Minimum fluid density (DENSEMIN), enter “1.0”.

- For Maximum fluid density (DENSEMAX), enter “1.026”.

This helps convergence, especially when starting with freshwater as the initial condition.

- In the Fluid density calculation section, for Fluid density calc (MT3DRHOFLG), enter “1”.

This indicates that the fluid density calculation will be done using the “Salt” species.

- For Reference fluid density (DENSEREF), enter “1.0”.

This means the reference fluid density (freshwater, in this case) at 25˚C is 1000 kg/m3.

- For Density/conc. slope (DRHODC), enter “0.7”.

This DRHODC value indicates that the density will vary linearly between 1.0 g/cm3 for freshwater and 1.026 g/cm3 for saltwater. DRHODC is estimated by dividing the density difference by the concentration difference. In this case, the result is the following:

| 1.026(g/cm3) - 1.0(g/cm3) | = 0.7 |
| --- | --- |
| 0.037(g/cm3) - 0.0(g/cm3) |  |

- Click OK to close the SEAWAT VDF Package dialog.

## Saving and Running SEAWAT

Now, save the changes and run SEAWAT:

- Save  the project.

- Click Run SEAWAT  to bring up the SEAWAT model wrapper dialog.

- When SEAWAT finishes, turn on Read solution on exit and Turn on contours (if not on already).

- Click Close to import the solution and close the SEAWAT dialog.

## Viewing the SS-1 Solution

Now, view the results of the SEAWAT model run:

- In the Project Explorer, under “ GC1 (MT3DMS)”, select “ Salt”.

- Click Contour Options  to bring up the Dataset Contour Options – 3D Grid – Salt dialog.

- In the Contour method section, from the first drop-down select “Linear”, and from the second drop-down select “Specify each color”.

- For Line thickness, enter “4”.

- In the Contour interval section, from the drop-down select “Specified Values”, and to the right enter “5”.

- In the spreadsheet, in row 1, for Start Value,  enter “0.01”.

- In row 2, for Start Value, enter “0.015”.

- In row 3, for Start Value, enter “0.02”.

- In row 4, for Start Value, enter “0.03”.

- In row 5, for Start Value, enter “0.035”.

- Click OK to close the Dataset Contour Options – 3D Grid – Salt dialog.

- Select time step 7 in the time step window.

The contours should appear similar to Figure 4. Feel free to review the other time steps as well.

Figure 4      Contours showing steady state concentration of salt at 30 minutes

Notice that the system reaches a steady state at around 30 mins, when the toe of the wedge has moved about 15.5 cm to the right.

### Flow Budget

Now check the water budget:

- In the Project Explorer, under “ GC1 (MODFLOW)” select “ Head”.

- In the Time Steps window, select time step 3.

- Using the Select j  tool, select the top left cell, then right-click and select Flow Budget… to bring up the Flow Budget dialog.

Notice that the Total Zone Flow in the Flow In column is about “83.14” (this is in cm3/min).

- When done, click OK to close the Flow Budget dialog.

- Save  the project.

# Second SEAWAT Scenario (SS-2)

## Saving the Model with a New Name

First, save the model with a new name:

- Select File | Save As… to bring up the Save As dialog.

- Select “Project Files (*.gpr)” from the Save as type drop-down.

- Enter “GC2.gpr” as the File name.

- Click Save to save the project under the new name and close the Save As dialog.

## Initial Conditions for SS-2

The MODFLOW heads simulated in the SS-1 scenario will be used as the initial head condition for the SS-2 scenario. The steady state salt concentrations simulated for SS-1 will be used as the initial concentration condition. The freshwater head on the right will be decreased to 26.2 cm to allow the wedge to intrude further into the system.

### Head Values

First, apply the head values to the grid.

- Under “ GC1 (MODFLOW)”, select “ Head”.

- Select MODFLOW | Global Options… to bring up the MODFLOW Global/Basic Package dialog.

- Turn off Starting heads equal grid top elevations.

- Click Starting Heads… to bring up the Starting Heads dialog.

- Click 3D Dataset → Grid… to bring up the Constant to Layer dialog.

- Select Apply to variable head cells only and click OK to bring up the Select Dataset dialog.

- In the Solution section, under “ GC1 (MODFLOW)”, select “ Head”.

- In the Time Steps window, select step 8 (value of “80.0”) and click OK to close the Select Dataset dialog.

- Click OK to close the Starting Heads dialog.

- Click OK to close the MODFLOW Global/Basic Package dialog.

### Salt Dataset Values

Next, adjust the salt dataset values for use in the new simulation.

- Under “ GC1 (MT3DMS)”, select “ Salt”.

- Select MT3DMS | Basic Transport Package… to bring up the Basic Transport Package dialog.

- In the spreadsheet in the bottom right, check the box in the Edit Per Cell column.

- In the Starting Conc. Per Cell column, click the  button to bring up the Starting Concentrations – Salt dialog.

- Click 3D Dataset → Grid… to bring up the Select Dataset dialog.

- In the Solution section, under “ GC1 (MT3DMS)”, select “ Salt”.

- In the Time Steps window, select step 17 (value of “80.0”) and click OK to close the Select Dataset dialog.

- Click OK to close the Starting Concentrations – Salt dialog.

- Click OK to close the Basic Transport Package dialog.

- Using the Select j  tool, select the top right cell in order to select the right column boundary cells.

- Right-click on one of the selected cells and select Properties… to bring up the 3D Grid Cell Properties dialog.

- On the MODFLOW tab, in the Starting head row, enter “26.2”.

- Click OK to close the 3D Grid Cell Properties dialog.

The SEAWAT global options and VDF Package settings do not need to be adjusted for this scenario.

## Saving and Running SEAWAT

- Save  the project.

- Click Run SEAWAT  to bring up the SEAWAT model wrapper dialog.

- When SEAWAT finishes, turn on Read solution on exit and Turn on contours (if not on already).

- Click Close to import the solution and close the SEAWAT dialog.

## Viewing the SS-2 Solution

- In the Project Explorer, under “ GC2 (MT3DMS)”, select “ Salt”.

- In the Time Steps window, select time step 2 (Figure 5).

Figure 5      Time step 2 (after 5 minutes)

- In the Time Steps window, select time step 4 (Figure 6).

Figure 6      Time step 4 (after 15 minutes)

- In the Time Steps window, select time step 12 (Figure 7).

Figure 7      Time step 12 (after 55 minutes)

The figures above match the data from the Goswami/Clement experiment (Figure 8).

Figure 8      Transient salt wedge intrusion profiles from Goswami/Clement experiment

The toe of the salt wedge predicted after 5, 15, and 55 minutes of simulation is at 22, 30, and 40 cm (respectively). It matches the data from the Goswami/Clement experiment. Compare the figures above to Figure 9, which shows the actual wedges from that experiment. The system is close to reaching steady-state conditions at about 60 to 80 minutes into the experiment.

Figure 9      Actual wedges from the Goswami/Clement experiment

- Save  the project.

# Third SEAWAT Scenario (SS-3)

## Saving the Model with a New Name

Now save the model with a new name.

- Select File | Save As… to bring up the Save As dialog.

- Select “Project Files (*.gpr)” from the Save as type drop-down.

- Enter “GC3.gpr” as the File name.

- Click Save to save the project under the new name and close the Save As dialog.

## Initial Conditions for SS-3

The MODFLOW heads simulated in the SS-2 scenario will be used as the initial head condition for the SS-3 scenario. The steady state salt concentrations simulated for SS-2 will be used as the initial concentration condition. The freshwater head on the right will be changed to 26.55 cm to allow the wedge to recede from the system.

### Apply Head Values

First, apply the head values to the grid:

- Under “ GC2 (MODFLOW)”, select “ Head”.

- Select MODFLOW | Global Options… to bring up the MODFLOW Global/Basic Package dialog.

- Turn off Starting heads equal grid top elevations.

- Click Starting Heads… to bring up the Starting Heads dialog.

- Click 3D Dataset → Grid… to bring up the Constant to Layer dialog.

- Select Apply to variable head cells only and click OK to bring up the Select Dataset dialog.

- In the Solution section, under “ GC2 (MODFLOW)”, select “ Head”.

- In the Time Steps window, select step 8 (value of “80.0”) and click OK to close the Select Dataset dialog.

- Click OK to close the Starting Heads dialog.

- Click OK to close the MODFLOW Global/Basic Package dialog.

### Salt Dataset Values

Next, adjust the salt dataset values for use in the new simulation.

- Under “ GC2 (MT3DMS)”, select “ Salt”.

- Select MT3DMS | Basic Transport Package… to bring up the Basic Transport Package dialog.

- In the spreadsheet in the bottom right, check the box in the Edit Per Cell column.

- In the Starting Conc. Per Cell column, click the  button to bring up the Starting Concentrations – Salt dialog.

- Click 3D Dataset → Grid… to bring up the Select Dataset dialog.

- In the Solution section, under “ GC2 (MT3DMS)”, select “ Salt”.

- In the Time Steps window, select step 17 (value of “80.0”) and click OK to close the Select Dataset dialog.

- Click OK to close the Starting Concentrations – Salt dialog.

- Click OK to close the Basic Transport Package dialog.

- Using the Select j  tool, select the top right cell in order to select the right column boundary cells.

- Right-click on one of the selected cells and select Properties… to bring up the 3D Grid Cell Properties dialog.

- On the MODFLOW tab in the Starting head row, enter “26.55”.

- Click OK to close the 3D Grid Cell Properties dialog.

The SEAWAT global options and VDF Package settings do not need to be adjusted for this scenario.

## Saving and Running SEAWAT

- Save  the project.

- Click Run SEAWAT  to bring up the SEAWAT model wrapper dialog.

- When SEAWAT finishes, turn on Read solution on exit and Turn on contours (if not on already).

- Click Close to import the solution and close the SEAWAT dialog.

## Viewing the SS-3 Solution

- In the Project Explorer, under “ GC3 (MT3DMS)”, select “ Salt”.

- In the Time Steps window, select time step 3 (Figure 10).

Figure 10      Time step 3 (after 10 minutes)

- In the Time Steps window, select time step 4 (Figure 11).

Figure 11      Time step 4 (after 15 minutes)

- In the Time Steps window, select time step 6 (Figure 12).

Figure 12      Time step 6 (after 25 minutes)

Notice that the figures above match the data from the Goswami/Clement experiment (Figure 13).

Figure 13      Transient salt wedge receding profiles from Goswami/Clement experiment

Compare the figures above to Figure 14, showing the actual receding wedges from that experiment. Notice that the wedge recedes much more quickly than it intrudes.

Figure 14      Actual receding wedges from the Goswami/Clement experiment

# Conclusion

This concludes the “SEAWAT – Goswami/Clement Experiment” tutorial. The following key concepts were discussed and demonstrated in this tutorial:

- SEAWAT combines MODFLOW and MT3DMS to solve variable density groundwater flow and solute transport problems.

- SEAWAT can be used to simulate the transient movement of transient saltwater wedges.

- Both model results and experimental data show that it takes more time to intrude than to recede.

## Further Reading

In addition to the Goswami and Clement (2007) article referenced in the tutorial, refer to the following journal articles to learn more about this topic. All the articles use SEAWAT to set up different types of experimental datasets involving density-coupled flows.

- Chang S.W., and T.P. Clement, (2013). “Laboratory and numerical investigation of transport processes occurring beneath a saltwater wedge” in Journal of Contaminant Hydrology, 147, pp. 14-24.

- Chang, S.W. and T.P. Clement, (2012). “Experimental and numerical investigation of saltwater intrusion dynamics in flux controlled groundwater systems” in Water Resources Research, Vol. 48, W09527.10 PP., doi:10.1029/2012WR012134, 2012.

- Goswami, R.R., T. P. Clement, and J. H. Hayworth, (2012). “Comparison of numerical techniques used for simulating variable-density flow and transport experiments” in ASCE Journal of Hydrologic Engineering, v.17 (2), pp.272-282.

- Chang, S. W, T. P. Clement, M. J. Simpson, and K. K Lee, (2011). “Does sea-level rise have an impact on saltwater intrusion?” in Advances in Water Resources, 34 (10), pp. 1283-1291.

Goswami, Rohit R. and Clement, T. Prabhakar, (2007). “Laboratory-scale investigation of saltwater intrusion dynamics” in Water Resources Research, Volume 43, W04418, doi:10.1029/2006WR005151, 2007. http://onlinelibrary.wiley.com/doi/10.1029/2006WR005151/abstract.

Goswami and Clement (2007), p. 1.

Goswami and Clement (2007), p. 6.

Goswami and Clement (2007), p. 8.

Goswami and Clement (2007), p. 5.

Goswami and Clement (2007), p. 8.