GMS 10.9

GMS 10.9 Tutorial

Geostatistics – 2D

Learn the various 2D interpolation methods available in GMS

Objectives

Learn to use the tools for manipulating scatter point sets and the interpolation schemes supported in GMS.

Time

30–45 minutes

Required Components

GMS Basic

Geostatistics

Prerequisite Tutorials

None

1 Introduction 2 1.1 Getting Started 2 2 Creating a Scatter Point Set 2 3 Importing a Scatter Point Set 3 3.1 Open the File 3 3.2 Import Wizard 3 3.3 Changing the Display Options 5 3.4 Creating a Bounding Grid 6 4 Selecting an Interpolation Scheme 7 4.1 Linear Interpolation 7 4.2 Simple IDW Interpolation 9 4.3 Kriging 14 5 Switching Datasets 16 6 Using the Data Calculator 16 7 Conclusion 17

# Introduction

Two-dimensional geostatistics (interpolation) can be performed in GMS using the 2D Scatter Point module. The module is used to interpolate from sets of 2D scatter points to any of the other object types (meshes, grids, TINs). Several interpolation schemes are supported, including kriging. Geostatistics are useful for setting up input data for analysis codes or for site characterization.

The tools for manipulating scatter point sets and the interpolation schemes supported in GMS are demonstrated and discussed in this tutorial. This includes how to create a scatter point set from scratch, how to import scatter points from a file, creating a bounding grid, interpolating the scatter points using various interpolation methods, and using the data calculator to compare interpolations. The interpolation schemes presented in this tutorial are defined in the Interpolation section of the GMS Online Help.

## Getting Started

Do the following to get started:

- If necessary, launch GMS.

- If GMS is already running, select the File | New command to ensure that the program settings are restored to their default state.

# Creating a Scatter Point Set

Interpolation in GMS is performed using scatter points. A set of 2D scatter points is defined by a set of xy coordinates. A group of scatter points is called a scatter point set. Each scatter point set may also have a dataset associated with it. Datasets are scalar or vector values—one value at each scatter point.

The scatter points represent the geometric objects while the datasets represent the data values at the points. The datasets may represent anything: elevation, heat measurements, concentration of a chemical, etc. The datasets can be interpolated to a TIN, mesh, or grid.

The Create Scatter Points  tool is used to create 2D scatter point sets in GMS by using the following steps:

- In the Project Explorer, right-click on the empty space and select New | 2D Scatter Point Set.

A new “ set (1)” scatter set will appear.

- Using the Create Scatter Points  tool, click several times in the Graphics Window to create a number of new scatter points.

When creating each point, GMS automatically assigns a dataset value at the point. The settings can be changed so that GMS gives a prompt for the dataset values.

- Select Scatter Points | Scatter Point Settings… to bring up the Preferences dialog.

Notice that “Scatter sets” is selected in the list on the left.

- In the 2D scatter section, turn on Confirm dataset value.

- Click OK to close the Preferences dialog.

- Click anywhere in the Graphics Window to create a new scatter point and bring up the Scatter Point Dataset Value dialog.

This dialog will appear each time a new scatter point is created to allow the value to be verified or changed.

- Click OK to exit the Scatter Point Dataset Value dialog.

The Create Scatter Points  tool only enters steady state data, which could also be imported from a file. To create a scatter point set with transient data, it must be imported from a file.

# Importing a Scatter Point Set

Scatter point sets can be imported from an existing file using the Text Import Wizard dialog. This dialog allows importing data into GMS from text files that are in columnar format. The file to be imported here was generated as an Excel spreadsheet and exported from Excel as tab-delimited text. The file contains scatter points which represent locations where the concentration of a contaminant has been estimated using a soil gas survey. The goal is to generate a map of the contaminant plume.

## Open the File

- Click New  to reset GMS.

- Select Don’t Save when prompted to save changes.

- Click Open  to bring up the Open dialog.

- Select “Text Files (*.txt,*.csv)” from the Files of type drop-down.

- Browse to the geos2d directory and select “plumedat.txt”.

- Click Open to close the Open dialog and bring up the Text Import Wizard – Step 1 of 2 dialog.

## Import Wizard

The first step of the Text Import Wizard dialog specifies how the data is delineated and where in the file the data should begin. For this file, the first row contains column headings. The first column contains the point labels, the second column contains the X values of the points, the third column contains the Y values, and the fourth column contains the dataset values for the points.

- Below the File import options section, turn on Heading row option (red arrow, Figure 1).

- Click Next> to go to the Step 2 of 2 page of the Text Import Wizard dialog.

Figure 1      Text Import Wizard with Heading row selected

This page of the Text Import Wizard allows specifying the meaning of the data in the file.

- From the GMS data type drop-down, select “2D Scatter Points”.

The data associated with 2D scatter points is now displayed in the File preview section. The next step is to tell GMS what each column in the file means. This is done by selecting the correct data type using the drop-downs in the Type row (the first row) of the spreadsheet for each column in the file. The options in the drop-downs change depending on the selected GMS data type. Since the use of a heading row was specified in the first step, GMS looked at the headings and automatically found and mapped the X and Y columns by selecting the X and Y selection in the Type row of the spreadsheet.

- In the first column, for the Type row, select “Label” from the drop-down.

- In the fourth column, for the Type row, select “Dataset” from the drop-down.

The dataset in this file represents concentrations of a contaminant. The No data option near the top of the dialog can be used to specify a key value in the file used to indicate a lack of information.

For example, if no measurement was taken at one of the points, entering “-999” as the No data value in the Text Import Wizard would tell GMS to ignore these points when doing the interpolation later. The key value can be any value that would not normally be encountered in the dataset. In this case, this option isn’t needed.

The dialog should now appear as in Figure 2.

- Click Finish to close the Text Import Wizard dialog.

A set of points should appear on the screen.

Figure 2      Text Import Wizard dialog - Step 2 of 2

## Changing the Display Options

Change the appearance of the scatter points using the Display Options dialog:

- Click Display Options  to bring up the Display Options dialog.

- Select “2D Scatter Data” from the list on the left.

- On the 2D Scatter Point Set tab, turn on Scatter point symbols and Contours.

- Click OK to exit the Display Options dialog.

Each of the points should now be displayed as a colored square (Figure 3). The color of the symbol represents the relative concentration of the contaminant at the point. When displaying colored symbols, it is useful to also display a color legend.

Figure 3      Scatter points with contour colors

- Click Display Options  to bring up the Display Options dialog.

- Select “ 2D Scatter Data” from the list on the left.

- On the 2D Scatter Point Set tab, to the right of Contours, click Options… to bring up the Dataset Contour Options – 2D Scatter Points - c dialog.

- At the bottom left, turn on Legend.

- Click OK to close the Dataset Contour Options – 2D Scatter Points - c dialog.

- Click OK to exit the Display Options dialog.

Notice that the concentrations vary from 0.0 to about 100.0 (see figure to right).

## Creating a Bounding Grid

The goal of this tutorial is to generate a series of contour plots illustrating the plume. It is first necessary to create a grid that bounds the scatter point set. This allows interpolating the concentrations from the scatter points to the grid nodes. The grid is then contoured.

- Right-click on “ plumedat” and select Bounding 2D Grid… to bring up the Create Finite Difference Grid dialog.

The x and y dimensions of this grid are already defined. The default values shown in the dialog cause the grid to extend beyond the scatter points by 10% on each side.

- In the X-Dimension section, for the Number of cells, enter “60”.

- In the Y-Dimension section, for the Number of cells, enter “40”.

- Click OK to close the Create Finite Difference Grid dialog.

- Frame  the project.

A grid should appear on the screen that encompasses only the scatter point set (Figure 4).

Figure 4      The grid encompassing the scatter points

# Selecting an Interpolation Scheme

Several interpolation schemes are supported in GMS because each has different strengths in different situations. Typically, the best approach is to try several schemes and then determine which scheme is giving the most reasonable results. GMS has been structured in such a way that multiple schemes can be tested quickly and easily.

## Linear Interpolation

Do the following to try simple linear interpolation:

- Select Interpolation | Interpolation Options… to bring up the 2D Interpolation Options dialog.

- In the Interpolation method section, select the Linear option.

- Click OK to close the 2D Interpolation Options dialog.

To interpolate to the grid, do the following:

- Select Interpolation | Interpolate → 2D Grid to bring up the Interpolate → Object dialog.

- Click OK to accept the defaults and close the Interpolate → Object dialog.

A series of contour lines will appear within the grid (Figure 5).

Figure 5      Contour lines after linear interpolation

### Viewing the Results

The contours show that the concentrations have been interpolated to the grid.

- Switch to Oblique View .

Notice that the grid has been deformed to match the contours (Figure 6). Color shading the entire surface can provide an even more effective display of the grid.

Figure 6      Oblique view

- Click Display Options  to bring up the Display Options dialog.

- Select “ 2D Grid Data” from the list on the left.

- On the 2D Grid tab, in the lower section, turn on Contours.

- Next to Contours, click Options... to bring up the Dataset Contour Options – 2D Grid – c_linear dialog.

- In the Contour method section, from the drop-down, select “Color Fill”.

- Click OK to close the Dataset Contour Options – 2D Grid – c_linear dialog.

- Click OK to exit the Display Options dialog.

The 2D grid should appear similar to Figure 7.

Figure 7      Linear interpolation with color fill

The outer part of the grid still has a concentration value equal to zero, as indicated by the flat, dark blue contour coloring. When linear interpolation is performed, the scatter points are triangulated to form a temporary TIN. A plane equation is computed for each triangle in the TIN and the coefficients of the plane equation are used to interpolate to points inside the triangle.

Therefore, linear interpolation cannot be performed for grid nodes outside the convex hull of the TIN (the boundary of the TIN). As a result, these nodes are assigned a value of zero. A value of zero is appropriate in this case since the concentrations of the scatter points on the perimeter of the scatter point set are zero.

### Viewing the Elliptical Control Function

The scatter points were generated from an elliptical control function. This shows how the different interpolation schemes compare to the original control function. To view the original control function:

- In the Project Explorer, right-click on “ grid” and select Import Dataset… to bring up the Dataset Filename dialog.

- Select “All Files (*.*)” from the Files of Type drop-down.

- Browse to the geos2d directory and select “ellipse2g.dat”.

- Click Open to import the dataset and close the Dataset Filename dialog.

- Select the “ control_function” dataset in the Project Explorer to make it active.

The Graphics Window should appear similar to Figure 8.

Figure 8      Interpolation scheme with original control function

## Simple IDW Interpolation

The next scheme to try is a simple form of inverse distance weighted (IDW) interpolation.

- In the Project Explorer, select “ 2D Scatter Data” to make it active.

- Select Interpolation | Interpolation Options… to bring up the 2D Interpolation Options dialog.

- In the Interpolation method section, select Inverse distance weighted.

- Next to Inverse distance weighted, click Options… to bring up the 2D IDW Interpolation Options dialog.

- In the Nodal function section, select Constant (Shepard’s method).

- Click OK to exit the 2D IDW Interpolation Options dialog.

- Click OK to exit the 2D Interpolation Options dialog.

- Select Interpolation | Interpolate → 2D Grid to bring up the Interpolate → Object dialog.

- Click OK to accept the defaults and close the Interpolate → Object dialog.

The Graphics Window should appear similar to Figure 9.

Figure 9      Inverse distance weighted (IDW) interpolation

The IDW scheme is a simple moving weighted averages scheme. To interpolate a value at a point, a weighted average of the nearby scatter points is used. The weights are an inverse function of distance. The closer a scatter point is to the interpolation point, the greater the weight given to the scatter point.

### IDW Interpolation with Gradient Planes

One of the problems with simple IDW interpolation is that the interpolated dataset always tends toward the mean of the dataset in the voids between scatter points. As a result, local minima or maxima in the voids in the scatter point set are not properly inferred. To overcome this problem, a nodal function can be computed at each scatter point.

A nodal function is a plane or quadratic function that is forced to pass through the scatter point and approximate the nearby scatter points in a least squares sense. When the interpolation is performed, rather than computing an average of the dataset values at the scatter point locations, an average is computed of the nodal functions of the nearby scatter point evaluated at the interpolation point. This approach allows local trends to be inferred and often results in a more accurate interpolation.

This is demonstrated through the following steps:

- In the Project Explorer, select “ 2D Scatter Data” to make it active.

- Select Interpolation | Interpolation Options… to bring up the 2D Interpolation Options dialog.

- In the Interpolation method section, next to Inverse distance weighted, click Options… to bring up the 2D IDW Interpolation Options dialog.

- In the Nodal function section, select the Gradient plane option.

- Click OK to exit the 2D IDW Interpolation Options dialog.

- Click OK to exit the 2D Interpolation Options dialog.

- Select Interpolation | Interpolate → 2D Grid to bring up the Interpolate → Object dialog.

- Click OK to accept the defaults and close the Interpolate → Object dialog.

The Graphics Window should appear similar to Figure 10.

Figure 10      IDW interpolation with gradient planar nodal functions

### Using the Horizontal Anisotropy Option

The interpolation can be stretched in a horizontal direction by using the horizontal anisotropy option in conjunction with a specified azimuth. One reason for using this option might be if it is known that a contaminant plume is elongated in a particular direction due to groundwater flow.

Do the following to change the horizontal anisotropy:

- Switch to Plan View .

- In the Project Explorer, select “ 2D Scatter Data” to make it active.

- Select Interpolation | Interpolation Options… to bring up the 2D Interpolation Options dialog.

- In the Anisotropy section, for the Horizontal anisotropy, enter “2.0”.

- For the Azimuth, enter “90.0”.

- Click OK to close the 2D Interpolation Options dialog.

- Select Interpolation | Interpolate → 2D Grid to bring up the Interpolate → Object dialog.

- For the Interpolated dataset name, enter “c_idw_grad_h2”.

- Click OK to close the Interpolate → Object dialog.

As can be seen in Figure 11, the data has been stretched horizontally. If desired, experiment with different values for the horizontal anisotropy.

Figure 11      Horizontal anisotropy

### IDW Interpolation with Quadratic Nodal Functions

The nodal functions used in IDW interpolation can also be quadratic functions which are constrained to pass through the scatter point and approximate the neighboring scatter points in a least-squares fashion. The averaging or blending of the quadratic functions during the interpolation stage often results in a very smooth surface.

- Switch to Oblique View .

- Select Interpolation | Interpolation Options… to bring up the 2D Interpolation Options dialog.

- In the Interpolation method section, next to Inverse distance weighted, click Options… to bring up the 2D IDW Interpolation Options dialog.

- In the Nodal function section, select the Quadratic option.

- Click OK to exit the 2D IDW Interpolation Options dialog.

- Click OK to exit the 2D Interpolation Options dialog.

- Select Interpolation | Interpolate → 2D Grid to bring up the Interpolate → Object dialog.

- Click OK to accept the defaults and close the Interpolate → Object dialog.

The Graphics Window should appear similar to Figure 12.

Figure 12      Quadratic nodal functions

### Truncation

Notice that the minimum value listed in the color legend is a negative number. This is impossible since there is no such thing as a negative concentration. By inferring trends, the nodal functions can sometimes project the plume values beyond zero and into the negative range. This type of error can be easily fixed using truncation.

- Select Interpolation | Interpolation Options… to bring up the 2D Interpolation Options dialog.

- In the lower right section, turn on Truncate values.

- Select Truncate to specified range.

- For Min, enter “0.0”.

- For Max, enter “150.0”.

- Click OK to close the 2D Interpolation Options dialog.

The concentrations should not go below zero, but it is okay to allow the interpolation scheme to infer a maximum concentration greater than the maximum measured value.

- Select Interpolation | Interpolate → 2D Grid to bring up the Interpolate → Object dialog.

- Enter “c_idw_quad_trunc” for the Interpolated dataset name.

- Click OK to close the Interpolate → Object dialog.

Notice that many of the concentrations around the perimeter of the map are now zero (Figure 13).

Figure 13      Truncated quadratic nodal functions

## Kriging

The last interpolation scheme to test is kriging. Kriging is based on the assumption that points near each other have a certain degree of spatial correlation, but points that are widely separated are statistically independent. Kriging is a set of linear regression routines that minimizes estimation variance from a predefined covariance model.

- Select Interpolation | Interpolation Options… to bring up the 2D Interpolation Options dialog.

- In the Interpolation method section, select Kriging.

- Next to Kriging, click Options… to bring up the Kriging Options dialog.

### Creating the Experimental Variogram

There are a large number of options to be specified in the Kriging Options dialog. While a variogram must always be defined, the defaults shown are adequate in most cases.

- Click Edit Variograms… to bring up the Variogram Editor dialog.

- In the Experimental variogram section, click the New button to bring up the Experimental Variogram dialog .

- Click OK to accept the defaults and close the Experimental Variogram dialog.

A curve should appear in the upper window of the Variogram Editor (Figure 14). This curve is called an experimental variogram. The experimental variogram is found by calculating the variance in dataset values of each scatter point in the set with respect to each of the other points and plotting the variances versus the distance between the points.

Figure 14      The experimental variogram curve in the Variogram Editor dialog

As can be seen in the plot of the experimental variogram, the shape of the variogram indicates that the variance is smaller at short separation distances. In other words, points that are close together have similar data values. The variance in the data values with many datasets becomes somewhat random after a certain level of separation. The variogram also oscillates around a value corresponding to the average variance. However, many of the points have zero values with concentration data and this tends to pull the experimental variogram back down.

### Creating the Model Variogram

Once the experimental variogram is computed, the next step is to define a model variogram. A model variogram is a simple mathematical function that models the trend in the experimental variogram. The model variogram is used in the kriging computations.

- In the Nested structure section of the Variogram Editor dialog, click the New button.

A red model variogram will appear in the variogram.

- From the Model function drop-down, select “Gaussian”.

The red variogram will change slightly.

- For Nugget, enter “25.0” in the text field.

- For Contribution, enter “1965.0” in the text field.

- For Range, enter “63.0” in the text field.

At this point, a reasonable fit should exist between the model variogram and the left half of the initial variogram. The right half is difficult to fit because of the zero values used in this case.

- Click OK to exit the Variogram Editor dialog.

- Click OK to exit the Kriging Options dialog.

- Click OK to exit the 2D Interpolation Options dialog.

- Select Interpolation | Interpolate → 2D Grid to bring up the Interpolate → Object dialog.

- Click OK to accept the defaults and close the Interpolate → Object dialog.

As seen in Figure 15, notice that this interpolation scheme results in an upward curvature towards the outside edges of the grid. This can be corrected by adding more scatter points with a concentration of “0.0” in these outlying areas.

Figure 15      Model variogram based on kriging interpolation

# Switching Datasets

Now that the grid has been interpolated using several different interpolation schemes, review the results by replotting some of the interpolated datasets. To do this, switch back to one of the previous datasets using the Project Explorer.

- Switch to Plan View .

- in the Project Explorer, in the “ 2D Grid Data” folder, under “ grid, select the “ c_linear” dataset”.

Use the Up and Down arrow keys on the keyboard to switch between the datasets. Notice the differences in the contours for each dataset.

# Using the Data Calculator

The Data Calculator  tool can be useful when comparing two datasets generated by interpolation. Use the Data Calculator to compute the difference between the kriging and IDW quadratic datasets by doing the following:

- Click Data Calculator  to bring up the Data Calculator dialog.

The available datasets are listed in the Datasets section, where they are numbered from “d1” to “d10”. Datasets are referenced in the mathematical expression using the “d” numbers. The “c_krig” dataset should be numbered “d10” and the “c_idw_quad” dataset should be labeled “d7”.

- In the Expression field, enter “abs(d7-d10)”.

- In the Result field, enter “Difference”.

- Click Compute to perform the calculation and create a new “Difference” dataset in the Project Explorer. It will also appear in the Datasets section as entry “d11”.

This computes the difference between the two datasets. Any new dataset computed using the Data Calculator is automatically designated the active dataset. With that calculation completed, it is helpful to view some basic statistics related to the new dataset.

- Click Dataset Info… to bring up the Dataset Info dialog.

This dialog displays basic statistics related to the selected dataset such as minimum, maximum, and mean data values. Feel free to review the statistics.

- Click OK to exit the Dataset Info dialog.

- Click Done to exit the Data Calculator dialog.

- Switch to Oblique View .

The contour plot now displayed represents the “ Difference” dataset that was just computed (Figure 16).

Figure 16      Difference contour in oblique view

# Conclusion

This concludes the “Geostatistics – 2D” tutorial. Key concepts demonstrated and discussed in this tutorial include the following:

- Interpolating from a sparse set of points to a different set of points (usually more dense and surface-defining points) by using 2D scatter points.

- 2D scatter points can be created manually or by importing them from a file.

- Several interpolation algorithms are available in GMS.

- Linear algorithms do not interpolate beyond the convex hull of the scatter points.

- Anisotropy and truncation can be used to help control the interpolation process.