GMS 10.9

GMS 10.9 Tutorial

HydroGeoSphere – Surface Flow and Evapotranspiration

Build a HydroGeoSphere model in GMS

Objectives

This tutorial demonstrates how to add surface flow and evapotranspiration to a HydroGeoSphere model in GMS.

Time

25–45 minutes

Required Components

GMS Core

HydroGeoSphere Model & Interface

Prerequisite Tutorials

Getting Started

HydroGeoSphere – Porous Media Domain

1 Introduction 2 2 Basic Workflow 3 3 Getting Started 4 3.1 Domains 5 4 Simulation Control 5 5 Zones 5 5.1 Surface Flow Domain 6 5.2 ET Domain 7 6 Material Properties 8 6.1 Surface Flow Domain 8 6.2 ET Domain 9 7 Boundary Conditions 10 7.1 Surface Flow Domain 10 7.2 ET 12 8 Save the Project and Simulation 12 9 Run Grok 13 10 Run Phgs 13 11 Read the Solution 13 12 Conclusion 13

# Introduction

This tutorial demonstrates how to build a HydroGeoSphere (HGS) model in GMS. It is based off the Intro to HydroGeoSphere tutorial from Aquanty. Refer to that tutorial and the HydroGeoSphere Reference Manual as needed.

In the Intro to HydroGeoSphere tutorial, the “R5 Catchment” model is developed. From that tutorial:

The model is quite simple compared to many ‘real world’ models that you may build. But it contains sufficient complexity that it is perfectly suited to demonstrate the overall HydroGeoSphere workflow. The intention of this tutorial is not to perfectly recreate the catchment, but to teach the process of creating a small watershed model in HGS. In fact, several of the inputs for the R5 model are completely arbitrary (e.g. the delineation of soil types and associated hydraulic properties, boundary condition values, etc.).

The R5 catchment covers a small (approximately 0.1 km2) rangeland catchment near Oklahoma City (Figure 3). The defined model includes basic flow in the porous medium and surface/overland flow domains, and includes evapotranspiration.

The HydroGeoSphere – Porous Media Domain tutorial covers creating a model of the porous media domain, and this tutorial adds the surface flow and evapotranspiration domains.

Figure 1     HydroGeoSphere Intro Tutorial, figure 3, pg. 14

# Basic Workflow

From the Intro to HydroGeoSphere tutorial, the basic HGS workflow consists of 5 steps:

- Data inputs are prepared (e.g. various GIS files, 2D grid/mesh files, etc.).

- Configure an input data file (the GROK file) for the model pre-processor.

- Use executables (*.exe) to generate the model input files and solve the simulation.

  - Grok.exe – compiles the GROK file, generates input files for the numerical simulator

  - Phgs.exe – reads model input files, performs the numerical simulation (i.e. it ‘solves’ the model)

  - Hsplot.exe OR hgs2vtu.exe – reformats model outputs for visualization

- Visualize, analyze, and interpret model results.

- Iterate on the model to add complexity, perform scenario analysis, calibrate, etc.

Note that a GROK file is prepared and then the grok.exe runs. During the run, grok.exe reads the GROK file and prepares the actual model input files. Only then can the model (phgs.exe) be run.

The HydroGeoSphere workflow is followed pretty closely when using GMS. The basic steps to build an HGS model in GMS—and how these steps relate to the HydroGeoSphere workflow—are as follows:

- Create the conceptual model (HGS step 1).

- Create a 3D UGrid for the porous media domain (HGS step 1).

- Create a 2D UGrid for the 2D domains: surface flow, ET (HGS step 1).

- Save the simulation, which creates the GROK file (HGS step 2).

- Run grok.exe on the GROK file to prepare input files for phgs.exe (HGS step 3a).

- Run phgs.exe (HGS step 3b).

- Read the solution and visualize the results (HGS step 4).

- Iterate on the model to add complexity, perform scenario analysis, calibrate, etc.… (HGS step 5, not covered in this tutorial).

# Getting Started

Do as follows to get started:

- If necessary, launch GMS.

- If GMS is already running, select File | New to ensure that the program settings are restored to their default state.

- Click Open  to bring up the Open dialog.

- Select “Project Files (*.gpr)” from the Files of type drop-down.

- Browse to the \hgs-olf_et\hgs-olf_et folder and select “start.gpr”.

- Click Open to import the project and exit the Open dialog.

The project should be visible in the Graphics Window (Figure 2). The project contains a 2D UGrid and a 3D UGrid. The porous media domain and initial simulation has already been defined.

Figure 2      The initial project

Before continuing, it is recommended to save the project with a new name.

Select File | Save As… to bring up the Save As dialog.

Browse to the directory for this tutorial.

Select “Project Files (*.gpr)” from the Save as type drop-down.

Enter “hgs_olf_et.gpr” as the File name.

Click Save to save the project and close the Save As dialog.

## Domains

HydroGeoSphere uses “domains” to describe different parts of the hydrosphere. There are several domains, but the only ones used in this tutorial include:

- Surface flow or overland flow (olf): fluid flow on the surface

- ET (et): evapotranspiration

Although the surface flow and ET domains are 2D in nature, HydroGeoSphere only uses one 3D grid. In GMS both a 3D grid and a 2D grid are used to help prepare the inputs and to display the results when the surface flow and/or ET domains are being modeled. The “ top 2d” UGrid will be used for the surface flow and ET domains.

- Turn the “ 3d” UGrid on and off to see that the “ top 2d” UGrid is simply a 2D UGrid at the top of the “ 3d” UGrid.

- When done, switch to Plan View .

# Simulation Control

Simulation parameters can now be assigned.

- Right-click the “ R5” simulation and select Simulation Control… to open the Simulation Control dialog.

Many HydroGeoSphere inputs are set in the Simulation Control dialog. Besides parameters that are traditionally associated with HydroGeoSphere simulation control, this dialog also includes time stepping, output, zones, and initial conditions.

- Select the General tab and turn on the following options:

- Surface flow

- ET

- Click OK to close the Simulation Control dialog.

# Zones

HydroGeoSphere uses the idea of “zones” to divide the model domain into different areas. Properties such as hydraulic conductivity are associated with one or more zones and thus can vary spatially. Different zones can be used for different domains (porous media domain, surface flow domain etc.).

In GMS, zones are defined using cell-based datasets. These datasets can be created in a variety of ways. Hydraulic conductivity and other properties are defined on “materials”, and materials are assigned to one or more zones.

This tutorial will demonstrate zone datasets for the surface flow and ET domains.

## Surface Flow Domain

The 2D UGrid is used for the surface flow, (or overland flow) domain, and the ET domain. For surface flow, there will be two zones. Most cells will be in zone 1 which will be assigned to a “Grassland” material. Cells around the roads will be assigned to zone 2 and assigned to a “Ranch Road” material.

- Turn off the “ 3d” UGrid.

- Select the “ top 2d” UGrid to make it active.

- Right-click on the “ top 2d” UGrid and select Z Values → Cell Dataset. A new cell-based dataset named “ elevation” will appear in the Project Explorer.

- Right-click on the “ elevation” dataset under “ top 2d” UGrid and select Rename.

- Enter “surface flow zones” as the new name.

- Right-click on the “ top 2d” UGrid and select Lock / Unlock Editing.

To assign values for zone 1 (“Grassland”),do the following:

- With the Select Cells  tool active, use Ctrl + A to select all cells.

- Change the dataset value S to “1.0”.

To assign values for zone 2 (“Ranch Road”),do the following:

- Remain in Plan View .

- Under the “ Map Data” folder, turn on and select the “ zones” coverage to make it active.

- Using the Select Polygons  tool, select the polygon representing the roads.

- Right-click and select the Select Intersecting Objects… command to open the Select Objects of Type dialog.

- Select the Active UGrid cells option and click OK to close the Select Objects of Type dialog.

- Change dataset value S to “2.0”.

- Deselect the polygon.

- Click the Contour Options  macro to open the Dataset Contour Options – UGrid – surface flow zones dialog.

- Change the Contour method to “Block Fill”.

- Click OK to close the Contour Options – UGrid – surface flow zones dialog.

- Click Yes at the message to turn on contours.

The surface flow zones dataset now has values of 2 along the roads, and a value of 1 everywhere else.

Figure 3      The surface flow dataset with assigned values

Set surface flow zones by doing the following:

- In the Project Explorer, double-click on the “ R5” simulation to open the Simulation Control dialog.

- Select the Zones tab.

- Turn on the Read surface flow zones from file option.

- Click the Select Dataset… button to open the Select Dataset dialog.

- Select the “ surface flow zones” dataset under the “ top 2d” UGrid.

- Click OK to close the Select Dataset dialog.

- Click OK to close the Simulation Control dialog.

## ET Domain

For ET, the zone encompasses all the top faces.

Start with creating the ET zones dataset by doing the following:

- Right-click the “ surface flow zones” dataset and select Duplicate.

- Right-click on the “ surface flow zones (2)” dataset and select Rename.

- Enter “et zones” as the new name.

- With the Select Cells  tool active, use Ctrl + A to select all cells.

- Change the dataset value S value to “1.0”.

Now set the ET zones by doing the following:

- Double-click on the “ R5” simulation in the Project Explorer to open the Simulation Control dialog.

- Select the Zones tab.

- Turn on the Read ET zones from file option.

- Click the Select Dataset… button to open the Select Dataset dialog.

- Select the “ et zones” dataset under the “ top 2d” UGrid.

- Click OK to close the Select Dataset dialog.

- Click OK to close the Simulation Control dialog.

# Material Properties

Properties such as hydraulic conductivity are associated with HydroGeoSphere materials. This idea is similar to traditional GMS materials, but HydroGeoSphere materials are completely separate from the traditional GMS materials. Multiple HydroGeoSphere materials can be defined, and materials are assigned to domains and to one or more zones. Note that if material property values are not assigned, then a default value will be applied. This example will create three materials. In this case, each material will be assigned to one zone, but a material can be assigned to multiple zones.

## Surface Flow Domain

Now define two materials with the surface flow domain.

- Right-click on the “ R5” simulation and select Materials… to open the Materials dialog.

### Create the “Grassland” Material

- Click Add Row  to add a new material.

- Change the new material name to “Grassland”.

- Change the Domain to “Surface flow” and ensure that Zones is set to “1”.

- Turn on X friction and enter “0.3”.

- Turn on Y friction and enter “0.3”.

- Turn on Rill storage height and enter “0.01”.

- Turn on Coupling length and enter “0.01”.

### Create the “Ranch Road” Material

- Click Add Row  to add a new material.

- Change the new material name to “Ranch Road”.

- Change the Domain to “Surface flow” and change Zones to “2”.

- Turn on X friction and enter “0.03”.

- Turn on Y friction and enter “0.03”.

- Turn on Rill storage height and enter “0.002”.

- Turn on Coupling length and enter “0.01”.

## ET Domain

Now define a material with the ET domain.

- Click Add Row  to add a new material.

- Change the new material name to “et1”.

- Change the Domain to “ET” and ensure the Zones is set to “1”.

- Turn on Evaporation depth and set it to “3”.

- Turn on EDF quadratic decay function.

- Turn on Root depth and set it to “3”.

- Turn on RDF quadratic decay function.

- Turn on LAI tables and click the Table… button to open the XY Series Editor dialog.

- Change the Number of rows to “2”.

- Enter the following into the table:

| time | lai |
| --- | --- |
| 0.0 | 2.08 |
| 1e41 | 2.08 |

- Click OK to close the XY Series Editor dialog.

- Turn on Transpiration fitting parameters and enter the following values:

  - C_1 set to “0.1”.

  - C_2 set to “0.05”.

  - C_3 set to “2.0”.

- Turn on Transpiration limiting pressure head and enter the following values:

  - Hwp_et set to “-150”.

  - Hfc_et set to “-3.8”.

  - Ho_et set to “0”.

  - Han_et set to “0”.

- Turn on Evaporation limiting pressure head and enter the following values:

  - He2_et set to “-1.5”.

  - He1_et set to “-0.5”.

- Turn on Canopy storage parameter and set the value to “0.0”.

- Turn on Initial interception storage and set the value to “0.0”.

All the materials have now been set.

- Click OK to exit the Materials dialog.

# Boundary Conditions

Boundary conditions are defined on feature objects in GMS coverages. When the model is saved, GMS intersects the features with the UGrid to find the grid components (nodes, cells, faces, or segments) that should be associated with the boundary conditions, and writes the boundary conditions to the GROK file.

## Surface Flow Domain

The surface flow domain includes critical depth boundary conditions on the boundary arcs and a rain boundary condition over all the top faces.

Start with creating a coverage that contains the outer boundary. This coverage will be reused for the surface flow and ET domains. To create the coverage:

- In the Project Explorer, turn off “ UGrid Data”.

- Right-click on the “ zones” coverage and select Duplicate.

- Right-click on “ Copy of zones” and select Rename.

- Enter “outer boundary” for the new name.

- Right-click on the “ outer boundary” coverage and select Coverage Type | HydroGeoSphere | Boundary Conditions.

- Using the Select Arc  tool, select the arc in the middle of the model that defines the clay lens area and press the Delete key.

- Repeat the previous step to delete the arc representing the area of the roads so that only the outer boundary arc remains.

- Select Feature Objects | Build Polygons.

To create the surface flow domain coverage:

- Right-click on the “ outer boundary” coverage and select Duplicate.

- Right-click on “ Copy of outer boundary” and select Rename.

- Enter “surface flow BCs” for the new name.

- In the Project Explorer, turn off all the coverages except for “ surface flow BCs”.

A boundary condition needs to be assigned to the top edge. To do this, separate the top edge to be a feature arc.

- Using the Select Vertices  tool, hold down the Shift key and select the top left and right corners.

- Right-click and select Vertex → Node.

- Using the Select Points\Nodes  tool, select the two nodes on the left side of the polygon.

- Right-click and select Node → Vertex.

To define the critical depth boundary conditions:

- Using the Select Arc  tool, double-click the top arc to open the Arc Properties dialog.

Different types of boundary conditions are listed on the left. Parameters for the boundary condition are shown on the right. Only one boundary condition can be assigned to each feature object.

- In the list on the left, turn on Critical depth.

- For the Name, enter “CritDepth_outlet”.

- Turn on Map to grid boundary.

- Click OK to close the Arc Properties dialog.

This tells GMS that the arc is to be intersected with the outer boundary of the grid and not the interior.

- Using the Select Arc  tool, double-click the bottom arc to open the Arc Properties dialog.

- In the list on the left, turn on Critical depth.

- For the Name enter “CritDepth_rest”.

- Turn on Map to grid boundary.

- Click OK to close the Arc Properties dialog.

To define the rain boundary condition:

- Using the Select Polygons  tool, double-click the polygon to open the Polygon Properties dialog.

- In the list on the left, turn on Rain.

- For the Name enter “Precip”.

- For the Constant value enter “5.555e-7”.

- Click OK to close the Polygon Properties dialog.

The boundary conditions are now defined for the surface flow domain but still need to be included in the “ R5” simulation.

- Right-click on the “ surface flow BCs” coverage and select Apply to | HydroGeoSphere Simulations → R5.

Figure 4      The HydroGeoSphere Simulation in the Project Explorer

## ET

Evapotranspiration will be assigned as a boundary condition on all the top faces.

- Right-click on the “ outer boundary” coverage and select Duplicate.

- Right-click on “ Copy of outer boundary” and select Rename.

- Enter “et BCs” for the new name.

- Using the Select Polygons  tool, double-click the polygon to open the Polygon Properties dialog.

- In the list on the left, turn on Potential evapotranspiration.

- For the Name enter “PET1”.

- For the Constant value enter “3e-8”.

- Click OK to close the Polygon Properties dialog.

The boundary conditions for the evapotranspiration have now been defined.

- Right-click on the “ et BCs” coverage and select Apply to | HydroGeoSphere Simulations → R5.

Figure 5      The HydroGeoSphere simulation in the Project Explorer

# Save the Project and Simulation

Before continuing, complete the following:

- Save  the project.

This saves the GMS project files but not the HydroGeoSphere simulation files. To create the files for the HydroGeoSphere simulation, the simulation needs to be saved.

- Under the “ R5” simulation, right-click on the “porous media BCs” coverage and select Remove.

- Right-click on the “ R5” simulation and select Save Simulation to open the Saving HydroGeoSphere Simulation dialog.

- When finished, click OK to close the Saving HydroGeoSphere Simulation dialog.

Saving the simulation creates the GROK file and the other files that it refers to. The data in the files is based on the parameters set for the simulation in GMS. To view these files:

- Right-click on the “ R5” simulation and select Open Containing Folder to open a Windows File Explorer.

The opened file explorer shows the directory where GMS saved the model files.

- In a separate Windows File Explorer dialog, browse to the \hgs-olf_et\hgs-olf_et folder.

- Drag the file “R5o.hgs_output_files.txt” file from the “hgs-olf_et” folder and drop it in the “R5” folder.

- Close both Windows File Explorer dialogs and return to GMS.

# Run Grok

- Right-click the “ R5” simulation and select the Run grok command to open a run dialog.

The “Run grok.exe” dialog appears. The output from grok.exe is displayed. It should only take a few seconds for grok.exe to finish. When it is done, there may be some warnings at the bottom of the output but there should not be any errors.

- When the process finishes, click OK.

# Run Phgs

Now that the input files have been prepared by running grok.exe, the model is ready to be run.

- Right-click the “ R5” simulation and select the Run phgs command.

The “Run phgs.exe” dialog appears. The progress dialog shows a plot of the residual error versus time, and the output from phgs.exe. It will take a few minutes for phgs.exe to finish.

- When the process finishes, click OK.

# Read the Solution

- Right-click the “ R5” simulation and select Read Solution to start the Read Solution progress dialog.

When GMS is done reading the solution, the Read Solution progress dialog should disappear automatically if there are no errors.

A new Solution folder was added under the R5 simulation containing links to datasets that were added under “R5 (HydroGeoSphere)” folders under the “3d” and “top 2d” UGrids. The HydroGeoSphere – Post-Processing tutorial has more information about how to examine the solution.

# Conclusion

This concludes the “HydroGeoSphere – Surface Flow and Evapotranspiration” tutorial.

Aquanty Guide for New Users, Intro to HydroGeoSphere Tutorial download at https://www.aquanty.com/s/HGS_Intro_Tutorial.zip

HydroGeoSphere Intro Tutorial, pg. 13

HydroGeoSphere Intro Tutorial, pg. 13