GMS 10.9

GMS 10.9 Tutorial

MODFLOW – Stochastic Modeling, Inverse

Use PEST to calibrate multiple MODFLOW simulations using material sets

Objectives

This tutorial will show how to use the stochastic inverse modeling option for MODFLOW and calibrate multiple MODFLOW models with equally probable “realizations” of the aquifer stratigraphy. This approach is demonstrated using the LPF and HUF packages.

Time

45–60 minutes

Required Components

GMS Core

MODFLOW Interface

PEST

Parallel PEST

Stochastic & T-PROGS

Prerequisite Tutorials

MODFLOW – Advanced PEST

MODFLOW – Stochastic Modeling, Indicator Simulations

1 Introduction 2 2 Getting Started 3 2.1 Importing the Project 3 2.2 Reviewing the MODFLOW Model Data 4 3 Selecting the Stochastic Option 5 3.1 Saving the Project and Running MODFLOW 5 3.2 Importing and Viewing the MODFLOW Solutions 6 4 Using PEST SVD Assist 7 4.1 Saving the Project and Running MODFLOW 8 4.2 Importing and Viewing the MODFLOW Solutions 8 4.3 Probabilistic Capture Zones 8 5 Stochastic Inverse with HUF 9 5.1 Importing and Viewing the MODFLOW Solutions 10 6 Conclusion 12

# Introduction

GMS supports three methods for performing stochastic simulations: parameter randomization, indicator simulations, and PEST Null Space Monte Carlo. This tutorial will use the indicator simulation approach in conjunction with PEST to create multiple calibrated MODFLOW models. The former approach is described in a separate tutorial.

The indicator simulation approach allows for generation of multiple, equally probable realizations of the aquifer stratigraphy. These realizations represent different distributions of material (indicator) zones within the aquifer. A set of aquifer properties is associated with the materials and the model is run once for each of the N realizations.

In GMS, the multiple realizations of the aquifer heterogeneity are typically generated using the T-PROGS software. T-PROGS can be used to generate two types of output: multiple material sets (arrays of material IDs), or multiple MODFLOW HUF input sets.  For this tutorial, a pre-defined set of material sets generated by T-PROGS will be used. The steps involved in running a T-PROGS simulation are described in the “T-PROGS” tutorial.

A groundwater model for a medium-sized basin is shown in Figure 1. The basin encompasses 72.5 square kilometers. It is in a semi-arid climate, with average annual precipitation of 0.381 m/yr. Most of this precipitation is lost through evapotranspiration. The recharge that reaches the aquifer eventually drains into a small stream at the center of the basin.

This stream drains to the north and eventually empties into a lake with an elevation of 304.8 m. Three wells in the basin extract water from the aquifer. The perimeter of the basin is bounded by low permeability crystalline rock. There are ten observation wells in the basin and a stream flow gauge at the bottom end of the stream.

This tutorial will discuss and demonstrate opening a MODFLOW model using the LPF package, running PEST in stochastic inverse mode, and running Parallel PEST with SVD Assist in stochastic inverse mode. It will then show how to view probabilistic capture zones, open a MODFLOW model using the HUF package, and running PEST in stochastic inverse mode.

Multiple realizations of the aquifer properties have been generated.

Figure 1      Sample model used in calibration exercise

# Getting Started

Do the following to get started:

- If necessary, launch GMS.

If GMS is already running, select File | New to ensure that the program settings are restored to their default state.

## Importing the Project

First, import a project containing the MODFLOW model and the material sets generated by T-PROGS:

- Click Open  to bring up the Open dialog.

- Select “Project Files (*.gpr)” from the Files of type drop-down.

Browse to the sto_inv_matset\sto_inv_matset\ directory and select “lpf.gpr”.

Click Open to import the project and exit the Open dialog.

A one layer MODFLOW model showing a four-material distribution should be visible (Figure 2).

Figure 2      The initial one layer MODFLOW model

Now view the different material sets generated by T-PROGS:

Fully expand the “ 3D Grid Data” folder in the Project Explorer:

Select the “ TPROGS 1” material set, then use the up and down arrow keys on the keyboard to cycle through the material sets.

## Reviewing the MODFLOW Model Data

Most of the MODFLOW data for the model (boundary conditions, well pumping rate, top and bottom elevations, etc.) has already been entered. Before continuing, review the MODFLOW data that are somewhat more unique to this type of simulation.

- Select MODFLOW | LPF – Layer Property Flow… to open the LPF Package dialog.

At the top of the dialog, notice that the Use material IDs option is selected for the Layer property entry method. This means that an array of K (hydraulic conductivity) values will not be entered, as is normally the case with MODFLOW. Instead, material IDs will be used to define the K values.

Click Material IDs… to open the Materials dialog.

This dialog illustrates the material IDs assigned to cells. These material IDs are inherited from the active material set generated by T-PROGS.

Click OK to exit the Materials dialog.

Click Material Properties… to open the Materials dialog.

This dialog is used to assign aquifer properties, including hydraulic conductivity, to each of the materials used by the model. Notice that a key value has been assigned in the Horizontal k (m/d) column for each material. Defined parameters are also being used with the materials. When the MODFLOW model is saved, GMS uses the array of material IDs, the list of material properties, and the parameters to automatically generate the array of K values required by MODFLOW.

Click OK to exit the Materials dialog.

Click OK to exit the LPF Package dialog.

Select MODFLOW | Parameters… to open the Parameters dialog.

Notice that the dialog lists four parameters that correspond to the four materials that are assigned to the model grid.

Click OK to exit the Parameters dialog.

# Selecting the Stochastic Option

Before running MODFLOW, turn on the appropriate stochastic simulation options. First, select the stochastic inverse run option:

- Select MODFLOW | Global Options… to open the MODFLOW Global/Basic Package dialog.

In the Run options section, select the Stochastic Inverse.

Click OK to exit the MODFLOW Global/Basic Package dialog.

Next, specify the use of the material set method (as opposed to HUF set) in the stochastic simulation. When choosing the material set option, specify the desired group (folder) of material sets to use. In this case, use the “TPROGS” folder that has only two simulations.

Select MODFLOW | Stochastic… to bring up the Stochastic Options dialog.

In the Simulation Method section, select Material sets.

Select “TPROGS” from the Material sets drop-down.

Click OK to exit the Stochastic Options dialog.

## Saving the Project and Running MODFLOW

Now save the project and run MODFLOW in stochastic mode.

- Select File | Save As… to bring up the Save As dialog.

- Select “Project Files (*.gpr)” from the Save as type drop-down.

Enter “lpf_sto.gpr” as the File name.

Click Save to save the project under the new name and close the Save As dialog.

Select MODFLOW | Run MODFLOW to bring up the MODFLOW/PEST Parameter Estimation dialog.

PEST and MODFLOW are now running in stochastic inverse mode. As each model run finishes, the spreadsheet on the lower right will indicate the number of PEST iterations, the model error, and the parameter values. Depending on the speed of the computer, the simulation may take several minutes to complete.

## Importing and Viewing the MODFLOW Solutions

Once all the MODFLOW runs are completed, import the solutions.

- Turn on Read solution on exit and Turn on contours (if not on already).

Click Close to close the MODFLOW/PEST Parameter Estimation dialog and bring up the Reading Stochastic Solutions dialog.

Click OK to close the Reading Stochastic Solutions dialog and import the solutions.

Fully expand the new “ lpf_sto (MODFLOW)(STO)” folder in the Project Explorer.

Select the “ lpf_sto001 (MODFLOW)” solution in the Project Explorer.

The model should appear similar to Figure 3.

Figure 3      Contours for the lpf_sto001 solution

Select the “ lpf_sto002 (MODFLOW)” solution in the Project Explorer.

Notice that the contours for each solution vary greatly according to the distribution of materials (compare Figure 3 and Figure 4). Notice also that the material set is updated to correspond to the material set used to generate each particular solution.

Figure 4      Contours for the lpf_sto002 solution

# Using PEST SVD Assist

Instead of specifying a single value for each material, it is possible to use pilot points to estimate the HK of each material. Specifying pilot points and undertaking a stochastic inverse model with early versions of PEST would take too much time due to the number of model runs required. However, SVD-Assist greatly reduces the number of required runs, and the process can be sped up further by using Parallel PEST.

First, assign pilot points to the parameters then turn on Parallel PEST and SVD Assist:

- Select MODFLOW | Parameters… to open the Parameters dialog.

From the “HK_Sand” parameter row, use the  drop-down in the Value column to select “<Pilot points>”.

Click the  button above the  drop-down on the “HK_Sand” parameter row to bring up the 2D Interpolation Options dialog.

Use the Interpolating from section in the Dataset drop-down to select “sand”.

Click OK to exit the 2D Interpolation Options dialog.

Repeat steps 2–5 for the other three parameters (“HK_Silt”, “HK_ClSilt”, and “HK_ClSand”), selecting the respective dataset material for step 4 (e.g. “sand” for HK_Sand).

Click OK to exit the Parameters dialog.

Select MODFLOW | Parameter Estimation… to open the PEST dialog.

In the Parallel PEST section, turn on Use Parallel PEST.

In the SVD options section, turn on Use SVD and Use SVD-Assist.

Select OK to exit the PEST dialog.

## Saving the Project and Running MODFLOW

Before running MODFLOW in stochastic mode, save the project:

- Select File | Save As… to bring up the Save As dialog.

- Select “Project Files (*.gpr)” from the Save as type drop-down.

Enter “lpf_sto1.gpr” as the File name.

Click Save to save the project under the new name and close the Save As dialog.

Click Run MODFLOW  to bring up the MODFLOW/PEST Parameter Estimation dialog.

Parallel PEST and MODFLOW now run in stochastic inverse mode. The time needed to finish this run will vary depending on the speed of the computer running it.

## Importing and Viewing the MODFLOW Solutions

Once all the MODFLOW runs are completed, import the solutions:

- Turn on Read solution on exit and Turn on Contours (if not on already).

Click Close to exit the MODFLOW/PEST Parameter Estimation dialog and bring up the Reading Stochastic Solutions dialog.

Click OK to import the solutions and close the Reading Stochastic Solutions dialog.

Fully expand the new “ lpf_sto1 (MODFLOW)(STO)” folder in the Project Explorer.

Notice that the observation targets for these new results are more similar than those in the previous stochastic inverse run.

## Probabilistic Capture Zones

Now import the results from a stochastic inverse run using the “TPROGS_A” material sets:

- Select MODFLOW | Read Solution… to bring up the Open dialog.

- From the drop-down box to the right of the File name field, select “MODFLOW Name File (*.mfn)”.

Browse to the sto_inv_matset\sto_inv_matset\run1_MODFLOW directory and select “run1.mfn”.

Click Open to exit the Open dialog and bring up the Reading Stochastic Solutions dialog.

Click OK to import all of the solutions and close the Reading Stochastic Solutions dialog.

Right-click on the new “ run1 (MODFLOW)(STO)” folder in the Project Explorer and select Risk Analysis… to bring up the Risk Analysis Wizard dialog.

Below the list field, select Probablistic capture zone analysis.

Click Next to go to the Capture Zone Analysis dialog.

Click Finish to run the analysis and close the Capture Zone Analysis dialog.

MODPATH is now running in the background. A progress bar should update as MODPATH is run for each of the simulations in the stochastic solution. When MODPATH is finished running, GMS will create a new dataset for each well in the model. The probability of capture from each cell to each well in the model can be viewed.

Expand “ Display Themes” in the Project Explorer and select “color fill contours”.

Contours similar to Figure 5 will now be visible showing the probabilistic capture zone for the well near the top of the model.

Figure 5      Probabilistic capture zone for a well

# Stochastic Inverse with HUF

The stochastic inverse approach can also be used with multiple HUF data sets. Now import a project with multiple HUF datasets and run a stochastic inverse model.

- Save  the current project.

Click New  to start a new project and reset to GMS defaults.

Click Open  to bring up the Open dialog.

Select “Project Files (*.gpr)” from the Files of type drop-down.

Browse to the sto_inv_matset\sto_inv_matset\ directory and select “huf.gpr”.

Click Open to open the project and exit the Open dialog.

Select “ 3D Grid Data” to make it active.

Select Edit | Select by ID… to bring up the Find Grid Cell dialog.

In the Cell ID field, enter “1452”.

Click OK to close the Find Grid Cell dialog.

Switch to Side View .

The model should appear similar to Figure 6. This model is the same as the one previously used. However, this model uses the HUF package instead of the LPF package. Notice the different hydrogeologic units defined in the HUF package now visible.

Figure 6      Hydrogeologic units from the HUF package

Select MODFLOW | Global Options… to open the MODFLOW Global/Basic Package dialog.

In the Run Options section, select the Stochastic Inverse option.

Click OK to exit the MODFLOW Global/Basic Package dialog.

Select MODFLOW | Stochastic… to bring up the Stochastic Options dialog.

In the Simulation Method section, select HUF sets and select “TPROGS” from the drop-down to the right.

Click OK to exit the Stochastic Options dialog.

Select File | Save As… to bring up the Save As dialog.

Select “Project Files (*.gpr)” from the Save as type drop-down.

Enter “huf_sto.gpr” as the File name.

Click Save to save the project under the new name and close the Save As dialog.

Select MODFLOW | Run MODFLOW to bring up the MODFLOW/PEST Parameter Estimation dialog.

Parallel PEST and MODFLOW are now running in stochastic inverse mode. The model run may take several minutes, depending on the speed of the computer being used.

## Importing and Viewing the MODFLOW Solutions

Once all the MODFLOW runs are completed, import the solutions.

- Turn on Read solution on exit and Turn on contours (if not on already).

Click Close to close the MODFLOW/PEST Parameter Estimation dialog and bring up the Reading Stochastic Solutions dialog.

Click OK to import all converged solutions and close the Reading Stochastic Solutions dialog.

Switch to Plan View .

Fully expand the “ 3D Grid Data” folder.

In the “ huf_sto (MODFLOW)(STO)” folder, select the “ huf_sto002 (MODFLOW)” solution.

The model should appear similar to Figure 7. Notice that the HUF data is updated to correspond to the HUF set used to generate that particular solution. Feel free to review the “ huf_sto001 (MODFLOW)” solution as well.

Figure 7      The final view of the model for the huf_sto002 solution

# Conclusion

This concludes the “MODFLOW – Stochastic Modeling, Inverse” tutorial. The following key concepts were discussed and demonstrated in this tutorial:

- Calibrating multiple models using the stochastic inverse modeling option.

- The stochastic inverse modeling approach supports material sets and HUF sets.

- Material sets and HUF sets can be created using TPROGS.

- The Risk Analysis Wizard can be used to do a probabilistic capture zone analysis.