GMS 10.9

GMS 10.9 Tutorial

MODFLOW 6 – Grid Approach

Build a MODFLOW 6 model using a 3D UGrid

Objectives

This tutorial describes the grid approach to MODFLOW 6 pre-processing.

Time

25–35 minutes

Required Components

GMS Core

MODFLOW-USG Model & Interface

Prerequisite Tutorials

Getting Started

1 Introduction 2 2 Getting Started 3 2.1 Creating the UGrid 3 3 Creating the MODFLOW 6 Simulation 4 4 Temporal Discretization 5 5 Constant Head Package 5 6 The NPF Package 6 6.1 Options 7 6.2 ICELLTYPE 7 6.3 K 7 6.4 K33 7 7 The Recharge Package 8 8 The Drain Package 8 9 The Well Package 10 10 Discretization 11 11 Initial Conditions 12 12 Output Control 12 13 Saving the Simulation 12 14 Running MODFLOW 13 15 Viewing the Solution 13 15.1 Changing Layers 14 15.2 Color Fill Contours and Color Legend 14 16 Conclusion 14

# Introduction

A simple MODFLOW 6 simulation can be built in GMS using the grid approach. The grid approach involves working directly with the 3D grid to apply sources/sinks and other model parameters on a cell-by-cell basis. The grid approach is useful for simple problems or academic exercises where cell-by-cell editing is necessary.

In GMS, multiple MODFLOW 6 simulations can be created in a single project. This makes creating and comparing variations of the same model simple.

This tutorial uses the first example model that is distributed with MODFLOW 6 (Figure 1). Three aquifers will be simulated using three layers in the computational grid. The grid covers a square region measuring 75000 feet by 75000 feet.

The grid will consist of 15 rows and 15 columns, each cell measuring 5000 feet by 5000 feet in plan view. For simplicity, the elevation of the top and bottom of each layer will be flat. The hydraulic conductivity values shown are for the horizontal direction. For the vertical direction, the tutorial will use some fraction of the horizontal hydraulic conductivity.

Flow into the system is due to infiltration from precipitation and is defined as recharge in the input. Flow out of the system is due to buried drain tubes, discharging wells (not shown on the diagram), and a lake, which is represented by a constant head boundary on the left. Starting heads will be set equal to zero, and a steady state solution will be computed.

Figure 1      Sample problem to be solved

This tutorial discusses and demonstrates the following:

- Setting up a MODFLOW 6 simulation

- Running MODFLOW 6

- Viewing the solution

# Getting Started

Do as follows to get started:

- If necessary, launch GMS.

- If GMS is already running, select File | New command to ensure that the program settings are restored to their default state.

## Creating the UGrid

The first step is to create the UGrid.

- Right-click in the Graphics Window or Project Explorer and select New | UGrid 3D… to open the New UGrid dialog.

- In the New UGrid dialog, in the X-Dimension and Y-Dimension sections, change Length to “75000”.

- In the X-Dimension and Y-Dimension sections, change Number cells to “15”.

- In the Z-Dimension section, change Number cells to “5”.

- Make certain all of the Origin fields are set to “0”.

- Click OK to close the New UGrid dialog.

A UGrid should be visible in the Graphics Window (Figure 2).

Figure 2      Initial project

# Creating the MODFLOW 6 Simulation

The next step is to initialize the MODFLOW6 simulation.

- In the Project Explorer, right-click on “ Project” and select New Simulation | MODFLOW 6 to bring up the New MODFLOW 6 Simulation dialog.

First, select the UGrid. If multiple UGrids were available, the selected one becomes associated with the simulation. Since this project only has one UGrid available, use that one.

- In the Select UGrid section, verify the box next to “ ugrid” is checked.

Now select the packages. The input to MODFLOW 6 is subdivided into packages. Packages can be selected when setting up the simulation. If needed, packages can also be added to the simulation later. For now, select the following packages:

- In the GWF – Groundwater Flow Model section, select the following:

- CHD – Constant Head

- DRN – Drain

- RCH – Recharge

- WEL – Well

- Click OK to exit the New MODFLOW 6 Simulation dialog.

A new simulation (“ sim”) will appear in the Project Explorer. Also in the Project Explorer, the different packages are listed under the simulation.

- Expand the new simulation “ sim” and the “flow” package under it.

Along with the optional packages that were selected, the standard TDIS, IMS, DIS, IC, NPF, and OC packages are listed. These are the default names given by GMS. The names of the packages can be changed if desired.

# Temporal Discretization

Review the temporal discretization for the simulation in the TDIS package. This package controls the timing for the simulation.

- In the Project Explorer, right-click on “TDIS” and select Open… to bring up the Temporal Discretization (TDIS) Package dialog.

- Turn on OPTIONS in the Sections area.

- Change the TIME_UNITS to “SECONDS” to match the example.

- Under PERIODDATA, change the PERLEN in the first row to “86400”.

- Click OK to close the Temporal Discretization (TDIS) Package dialog.

# Constant Head Package

The next step is to define the head boundary along the west sides of the model. In some cases, it is easier to assign values directly to cells. This can be accomplished by editing the properties of the cells. Before using the command, it is necessary to first select the cells in the leftmost column.

Do the following to select the cells:

- Select the Select Cells  tool.

- Drag a box around all of the cells in the leftmost column of the grid to highlight them (Figure 3).

Figure 3      Highlight the leftmost column

To edit the head value:

- Right-click on one of the selected cells and select Cell Properties… to bring up the Cell Properties Dialog.

- Select “CHD” from the Packages list on the left.

- Click the Add Rows  button to open the Add Stresses dialog.

- On the HEAD row, in the Value column, enter “0.0”.

- Click OK to exit the Add Stresses dialog.

- Click OK to close the Cell Properties Dialog.

Notice that a diamond symbol is displayed in the cells that were edited, indicating they are constant head cells (Figure 4).

Figure 4      Constant head cells marked on the left

# The NPF Package

The next step is to enter the data for the Node Property Flow (NPF) package. The NPF package computes the conductance between each of the grid cells and sets up the finite difference equations for the cell-to-cell flow.

To enter the NPF data:

- In the Project Explorer, right-click on “NPF” and select Open… to bring up the Node Property Flow (NPF) Package dialog.

The options in the Node Property Flow (NPF) Package are used to define the cell type, hydraulic conductivity, and wetting and drying options for every cell in the model. This problem has five layers. The top layer is convertible, and the other layers are confined.

## Options

Start by adjusting the options:

- In the Sections list on the left, select OPTIONS.

- Turn on the VARIABLECV option.

- Turn on the DEWATERED option.

- Turn on the PERCHED option.

These options adjust how vertical conductance and head are calculated.

## ICELLTYPE

Now, enter the data for the ICELLTYPE:

- If necessary, select GRIDDATA from the Sections list on the left, and deselect OPTIONS.

- Select the ICELLTYPE tab.

- Turn on Layered and make certain it is set to layer “1”.

- For Constant, enter “1”.

- Change the layer to “2”.

- For Constant, enter “0”.

- Similarly set the remaining layers (3–5) to 0.

## K

Next, enter the data for the hydraulic conductivity (K):

- Select the K tab.

- Turn on Layered and make certain it is set to layer “1”.

- For Constant, enter “1e-3”.

- Change the layer to layer “2”.

- For Constant, enter “1e-8”.

- Change the layer to “3”.

- For Constant, enter “1e-4”.

- Change the layer to “4”.

- For Constant, enter “5e-7”.

- Change the layer to “5”.

- For Constant, enter “2e-4”.

## K33

Finally, enter the data for vertical hydraulic conductivity (K33):

- Select the K33 tab.

- Turn on the Define option.

- Turn on Layered and make certain it is set to layer “1”.

- For Constant, enter “1e-3”.

- Change the layer to “2”.

- For Constant, enter “1e-8”.

- Change the layer to “3”.

- For Constant, enter “1e-4”.

- Change the layer to “4”.

- For Constant, enter “5e-7”.

- Change the layer to “5”.

- For Constant, enter “2e-4”.

- Click OK to close the Node Property Flow (NPF) Package dialog.

# The Recharge Package

Entering data in the recharge package allows simulation of the recharge to an aquifer due to rainfall and infiltration. The Recharge and Evapotranspiration packages can use lists or arrays to define the data, unless the DISU package is used, in which case only lists can be used. The READASARRAY option in the Options section switches between the two modes.

To enter the recharge data, do the following:

- In the Project Explorer, right-click on “RCH” and select Open… to bring up the Recharge (RCH) Package dialog.

- If necessary, select PERIODS from the Sections list on the left.

- Select the RECHARGE tab.

- For Constant, enter “3e-8”.

- Click OK to exit the Recharge (RCH) Package dialog.

# The Drain Package

The next step is to define the row of drains in the top layer of the model. To define the drains, first select the cells where the drains will be located, and then select the Cell Properties… command.

The drains are located in the top layer (Figure 5).

Figure 5      Cells to be selected

To select the cells, do the following:

- In the UGrid Single Layer toolbar, select the Single layer checkbox.

- Enter “1” for the Layer then press the Tab key.

Figure 6      UGrid Single Layer toolbar

- Using the Select Cells  tool, draw a box around cells 2–10 on row 8 (Figure 5).

To assign drains to the cells:

- Right-click on one of the selected cells and select Cell Properties… to bring up the Cell Properties Dialog.

- Select “DRN” from the Packages list on the left.

- Click the Add Rows  button to open the Add Stresses dialog.

- Accept the defaults and click OK to exit the Add Stresses dialog.

At this point, enter an elevation and a conductance for the selected drains. The drains all have the same conductance, but the elevations are not all the same.

- Adjust the values in the ELEV column in the list of drains so the values match this table. All other values should already match the table:

| LAY | ROW | COL | ELEV | COND |
| --- | --- | --- | --- | --- |
| 1 | 8 | 2 | 0.0 | 1.0 |
| 1 | 8 | 3 | 0.0 | 1.0 |
| 1 | 8 | 4 | 10.0 | 1.0 |
| 1 | 8 | 5 | 20.0 | 1.0 |
| 1 | 8 | 6 | 30.0 | 1.0 |
| 1 | 8 | 7 | 50.0 | 1.0 |
| 1 | 8 | 8 | 70.0 | 1.0 |
| 1 | 8 | 9 | 90.0 | 1.0 |
| 1 | 8 | 10 | 100.0 | 1.0 |

- Click OK to close the Cell Properties Dialog.

- Deselect the cells by clicking anywhere outside the grid.

The drains should now be marked by circles (Figure 7).

Figure 7      Location of drain cells (green circles)

# The Well Package

Several wells need to be defined. Most of the wells are in the top layer but some are in the middle and bottom layers. Wells can be defined by selecting cells and entering data as was done with the drain package. However, for this exercise, define all the wells directly using the Well (WEL) Package dialog.

- In the Project Explorer, right-click on “WEL” and select Open… to bring up the Well (WEL) Package dialog.

- Click the Add Rows  button to open the Add Stresses dialog.

- Change the Number of rows to add to be “15”.

- Click OK to close the Add Stresses dialog.

- Define the wells using the values in the following table:

| LAY | ROW | COL | Q |
| --- | --- | --- | --- |
| 5 | 5 | 11 | -5 |
| 3 | 4 | 6 | -5 |
| 3 | 6 | 12 | -5 |
| 1 | 9 | 8 | -5 |
| 1 | 9 | 10 | -5 |
| 1 | 9 | 12 | -5 |
| 1 | 9 | 14 | -5 |
| 1 | 11 | 8 | -5 |
| 1 | 11 | 10 | -5 |
| 1 | 11 | 12 | -5 |
| 1 | 11 | 14 | -5 |
| 1 | 13 | 8 | -5 |
| 1 | 13 | 10 | -5 |
| 1 | 13 | 12 | -5 |
| 1 | 13 | 14 | -5 |

A copy of this table can be found as a spreadsheet file, named “wellpackage.xls”, included with the tutorial files. This table can be copied into the dialog using the copy (Ctrl+C) and paste (Ctrl+V) commands.

- Click OK to close the Well (WEL) Package dialog.

Now that all of the wells have been defined, it is possible to see them in the Graphics Window (Figure 8).

Figure 8      Wells added to the simulation

# Discretization

Discretization information is set through the DIS package:

- In the Project Explorer, right-click on “DIS” and select Open… to bring up the Structured Discretization (DIS) Package dialog.

- If necessary, select “GRIDDATA” from the Sections list on the left.

- Select the TOP tab.

- For Constant, enter “200”.

- Select the BOTM tab.

- Verify that the Layered option is on and set to layer “1”.

- For Constant, enter “-150”.

- Change the layer to “2”.

- For Constant, enter “-200”.

- Change the layer to “3”.

- For Constant, enter “-300”.

- Change the layer to “4”.

- For Constant, enter “-350”.

- Change the layer to “5”.

- For Constant, enter “-450”.

- Click OK to close the Structured Discretization (DIS) Package dialog.

# Initial Conditions

Review the initial conditions before running the simulation. To do this:

- In the Project Explorer, right-click on “IC” and select Open… to bring up the Initial Conditions (IC) Package dialog.

- Verify that Set starting heads equal to cell top elevations is turned on.

- Click OK to close the Initial Conditions (IC) Package dialog.

# Output Control

Before running the simulation, the output needs to be defined. To do this:

- In the Project Explorer, right-click on “OC” and select Open… to bring up the Output Control (OC) Dialog.

- Verify that the Preset output is set to “At every time step”.

- Click OK to close the Output Control (OC) Dialog.

This dialog also contains an Options section. There are two options for the budget file output and the head file output. The BUDGET FILEOUT option specifies a file to which desired flow terms can be written. The HEAD FILEOUT option specifies a file for head information.

# Saving the Simulation

At this point, the MODFLOW 6 data is completely defined and ready for the simulation run. Before running MODFLOW 6, save the simulation as a new project:

- Select File | Save As… to bring up the Save As dialog.

- Browse to the mf6_grid directory.

- Select “Project Files (*.gpr)” from the Save as type drop-down.

- Enter “mf6.gpr” as the File name.

- Click Save to save the project and close the Save As dialog.

# Running MODFLOW

It is now possible to run MODFLOW:

- In the Project Explorer, right-click on “ sim” and select Save Simulation.

- In the Project Explorer, right-click on “ sim” and select Run Simulation to bring up the Simulation Run Queue model wrapper dialog.

The Save Simulation command exports all of the MODFLOW 6 files necessary for the model run. The Simulation Run Queue shows all simulation model runs currently in progress. Since this project only has one simulation, only one is shown.

- When MODFLOW 6 finishes, click Load Solution.

- Click Close to exit the Simulation Run Queue dialog.

- Make sure the “ Head” dataset is active in the Project Explorer.

- Click Display Options  to bring up the Display Options dialog.

- Select “UGrid: ugrid – [Active]” from the list on the left.

- Turn on Define UGrid specific options.

- Turn on Face contours and click Options… to open the Dataset Contour Options – UGrid – Head dialog.

- Change the Contour Method to “Block Fill”.

- Click OK to close the Dataset Contour Options – UGrid – Head dialog.

- Click OK to close the Display Options dialog.

# Viewing the Solution

After the solution has been loaded and contours set, the Graphics Window should appear as in Figure 9.

Figure 9      Contours of MODFLOW 6 solution

## Changing Layers

View the solutions on the middle and bottom layers by doing the following:

- In the UGrid Single Layer toolbar, click the up arrow  to go to layer 3 (two clicks) or layer 5 (four clicks).

- When finished viewing the middle and bottom layer solutions, use the down arrow  to return to layer 1.

## Color Fill Contours and Color Legend

It is also possible to display the contours using a color fill option.

- Select Display | Contour Options… to bring up the Dataset Contour Options – UGrid – Head dialog.

- In the Contour method section, select “Color Fill” from the top drop-down.

- At the bottom left, turn on Legend if it is not already on.

- Click OK to close the Dataset Contour Options – UGrid – Head dialog.

The Graphics Window should appear similar to Figure 10.

Figure 10      Color fill contours with a legend

# Conclusion

This concludes the “MODFLOW 6 – Grid Approach” tutorial. The following topics were discussed and demonstrated:

- Creating a MODFLOW 6 simulation.

- Assigning simulation properties to cells in a UGrid.

- Defining package parameters.

- Running the simulation and using the Simulation Run Queue.