GMS 10.9

GMS 10.9 Tutorial

Stratigraphy Modeling – Horizons with Rasters

Create UGrids from rasters using the 3D UGrid from Rasters tool

Objectives

Learn how to create subsurface models using the 3D UGrid from Rasters tool.

Time

20–30 minutes

Required Components

GMS Core

Subsurface

Geostatistics

Prerequisite Tutorials

Stratigraphy Modeling – Horizons and Solids

1 Introduction 2 1.1 Getting Started 2 2 Importing the Project 2 3 Viewing Raster Elevations 3 4 Using the 3D UGrid from Rasters Tool 4 4.1 Viewing the UGrid 5 5 Changing the Fill and Clip Fields 6 5.1 Viewing the UGrid 6 5.2 Changing the Clip Field 7 5.3 Viewing the UGrid 8 6 Sacramento Data 8 6.1 Creating the UGrid 9 6.2 Viewing the UGrid 10 7 Conclusion 11

# Introduction

This tutorial builds on the concepts taught in the “Stratigraphy Modeling—Horizons and Solids” tutorial. In that tutorial, solids were created using horizons defined on boreholes and cross sections. In this tutorial, horizons will be assigned to rasters and then used to create 3D unstructured grids (UGrids). This tutorial will begin with a simple problem to illustrate the concepts involved in using rasters to create UGrids. Then this tutorial will apply the method to data from Sacramento, California, USA.

This tutorial will demonstrate the following:

- Importing a project with rasters.

- Creating a raster catalog and assign horizon IDs.

- Creating UGrids from the horizons.

- Applying these steps to data from the Sacramento, California region.

## Getting Started

Do the following to get started:

If necessary, launch GMS.

If GMS is already running, select the File | New command to ensure that the program settings are restored to their default state.

# Importing the Project

The first step in the construction of the UGrid is to import a set of borehole logs. Borehole data can be entered into GMS manually, or the data can be read from a file. In the interest of time, import a previously prepared file by doing the following:

- Click Open  to bring up the Open dialog.

Select “Project Files (*.gpr)” from the Files of type drop-down.

Browse to the Horizons_with_Rasters\Horizons_with_Rasters directory and select “rasters.gpr”.

Click Open to import the project and close the Open dialog.

A 2D UGrid and multiple rasters have been loaded into this GMS project (Figure 1).

Figure 1      Tin and rasters displayed in the Graphics Window

# Viewing Raster Elevations

The display options are currently set to view the rasters as points.

- Switch to Oblique View .

Frame  the project.

The two lower rasters slope upward toward the right of the image. The upper two rasters are at constant elevations (Figure 2).

Figure 2      Raster surfaces

The next step is to change the display of the raster to background imagery.

Change to Plan View .

In the Project Explorer, select the first listed unit  then hold the Shift key and select the last listed unit , to select all of the rasters.

Right-click on any of the rasters in the Project Explorer and select Display Options… to bring up the Raster Display Options dialog.

In the All Rasters section, select Display as 2D image and click OK to exit the Raster Display Options dialog.

In the Project Explorer, hide all of the rasters except “ unit1”.

The Graphics Window should now appear similar to Figure 3.

When done, turn on the other rasters in the Project Explorer.

Figure 3      Rasters displayed as background imagery

# Using the 3D UGrid from Rasters Tool

The 3D UGrid from Rasters tool creates a 3D UGrid using one or more rasters contained in the project. With the rasters imported in the project, they can now be used to generate 3D UGrid.

- Click the Toolbox   macro to open the Toolbox dialog.

- In the Tools tab, expand the “ Unstructured Grids” folder.

- Double-click on the 3D UGrid from Rasters tool to open the 3D UGrid from Rasters dialog.

- For the 2D UGrid drop-down, make certain “UGrid Data/ugrid (2)” is selected.

- In the Rasters section, click the Add Row  button five times so there are five rows.

- Using the following table, select the correct raster that matches the Horizon ID:

| Horizon ID | Raster | Fill | Clip |
| --- | --- | --- | --- |
| 5 | unit4 | √ |  |
| 4 | unit3 | √ |  |
| 3 | unit2 | √ |  |
| 2 | unit1 | √ |  |
| 1 | bottom | √ |  |

For the 3D UGrid name, enter “3d ugrid”.

Select OK to run the 3D UGrid from Raster dialog.

When the tool finishes, click OK to close the 3D UGrid from Raster dialog.

## Viewing the UGrid

The 3D UGrid from Rasters process should complete quickly, and a new UGrid will appear in the Project Explorer.

- Switch to Oblique View .

- In the Project Explorer, under the “ 3d ugrid”, select “ Cell Bottom Z”.

- Turn off “ ugrid (2)”.

The 3D Ugrid should look similar to Figure 4. Notice how the lower two layers extend upward on the left side of the UGrid and cut off the upper two layers.

Figure 4      UGrid created from raster catalog

# Changing the Fill and Clip Fields

Now see the effect of changing the Fill and Clip fields in the raster catalog. When the raster catalog was first created and the dialog appeared, the check boxes for the Fill field were on by default. The Fill field indicates to use the raster to create a UGrid layer. It means the 3D UGrid from Rasters operation will Fill between this raster and the lower surfaces. If the Fill field is turned off, then it will not create a layer that is associated with the particular raster. The Clip field is used to indicate a surface that “Clips” or truncates any lower surfaces. This process is best illustrated by the following steps:

- Click the Toolbox   macro to open the Toolbox dialog.

- In the Tools tab, expand the “ Unstructured Grids” folder.

- Double-click on the 3D UGrid from Rasters tool to open the 3D UGrid from Rasters dialog.

- For the 2D UGrid drop-down, make certain “UGrid Data/ugrid (2)” is selected.

- In the Rasters section, click the Add Row  button five times so there are five rows.

- Using the table below, select the correct raster that matches the Horizon ID and turn off the Fill for the “ unit3” raster:

| Horizon ID | Raster | Fill | Clip |
| --- | --- | --- | --- |
| 5 | unit4 | √ |  |
| 4 | unit3 |  |  |
| 3 | unit2 | √ |  |
| 2 | unit1 | √ |  |
| 1 | bottom | √ |  |

- For the 3D UGrid name, enter “3d ugrid fill”.

- Select OK to run the 3D UGrid from Raster dialog.

- When the tool finishes, click OK to close the 3D UGrid from Raster dialog.

## Viewing the UGrid

When the 3D UGrid from Rasters process is finished, another UGrid will have been created. To view the new UGrid, do the following:

- In the Project Explorer, hide “ 3d ugrid”.

- Under the “ 3d ugrid fill”, select “ Cell Bottom Z”.

The UGrid should look like Figure 5 below. Notice that no layer was created for “unit3”.

Figure 5      UGrid created with the “Fill” field off for the unit3 raster

## Changing the Clip Field

Now it is possible to change the Clip field for the “ unit3” raster and view the effect.

- Click the Toolbox   macro to open the Toolbox dialog.

- In the Tools tab, expand the “ Unstructured Grids” folder.

- Double-click on the 3D UGrid from Rasters tool to open the 3D UGrid from Rasters dialog.

- For the 2D UGrid drop-down, make certain “UGrid Data/ugrid (2)” is selected.

- In the Rasters section, click the Add Row  button five times so there are five rows.

- Using the table below, select the correct raster that matches the Horizon ID and change the Fill and Clip for the “ unit3” raster:

| Horizon ID | Raster | Fill | Clip |
| --- | --- | --- | --- |
| 5 | unit4 | √ |  |
| 4 | unit3 |  | √ |
| 3 | unit2 | √ |  |
| 2 | unit1 | √ |  |
| 1 | bottom | √ |  |

- For the 3D UGrid name, enter “3d ugrid clip”.

- Select OK to run the 3D UGrid from Raster dialog.

- When the tool finishes, click OK to close the 3D UGrid from Raster dialog.

## Viewing the UGrid

- In the Project Explorer, hide “ 3d ugrid fill”.

Under the “ 3d ugrid clip”, select “ Cell Bottom Z”.

The UGrid should look like Figure 6 below. Notice that while no layer was created for unit3, the raster was used to "clip" or truncate the units as seen in Figure 6.

Figure 6      UGrid created with the “Clip” field on for the unit3 raster

# Sacramento Data

It is now possible to use what has been learned to create a UGrid using data from the Sacramento Valley.

- Select the File | New command.

Select Don’t Save when asked to save the project.

Select the Open  button to bring up the Open dialog.

Select “Project Files (*.gpr)” from the Files of type drop-down.

Browse to the directory entitled Horizons_with_Rasters\Horizons_with_Rasters\ and select the file named “sacramento.gpr”.

Click Open to import the project and close the Open dialog.

The Figure 7 image should be visible in the GMS graphics window. This project contains two TINs to define the top and bottom of the UGrid, a map coverage with the surface geology, multiple rasters to define top elevations for stratigraphic units, and an aerial photo of the Sacramento region.

Figure 7      Map view of Sacramento Region Data

## Creating the UGrid

It is now possible to convert the rasters to a UGrid by doing the following:

- Click the Toolbox   macro to open the Toolbox dialog.

In the Tools tab, expand the “ Unstructured Grids” folder.

Double-click on the 3D UGrid from Rasters tool to open the 3D UGrid from Rasters dialog.

For the 2D UGrid drop-down, make certain “UGrid Data/ugrid” is selected.

In the Rasters section, click the Add Row  button seven times so there are seven rows.

Using the following table, select the correct raster that matches the Horizon ID.

| Horizon ID | Raster | Fill | Clip |
| --- | --- | --- | --- |
| 7 | sacramentotop | √ |  |
| 6 | TOP7.bil | √ |  |
| 5 | TOP6.bil | √ |  |
| 4 | TOP4.bil | √ |  |
| 3 | TOP2.bil | √ |  |
| 2 | TOP1.bil | √ |  |
| 1 | sacramentobottom | √ |  |

For the Minimum layer thickness option, enter a value of “1.0”.

For the 3D UGrid name, enter “3d ugrid”.

Select OK to run the 3D UGrid from Raster dialog.

When the tool finishes, click OK to close the 3D UGrid from Raster dialog.

## Viewing the UGrid

When the 3D UGrid from Rasters process finishes, a new UGrid will appear in the Project Explorer.

- Switch to Oblique View .

In the Project Explorer, under the “ 3d ugrid”, select “ Cell Bottom Z”.

Click the Display Options  macro to open the Display Options dialog.

From the list on the left, select “UGrid: 3d ugrid – [Active]”.

Check on Define UGrid specific options.

Turn off the Cell edge option and turn on the Face contours option.

Click the Options button to open the Dataset Contour Options – UGrid – Cell Bottom Z dialog.

Change the Contour method to “Color Fill”

Click OK to close the Dataset Contour Options dialog.

Click OK to close the Display Options dialog.

The UGrid should appear like Figure 8 below.

Figure 8      UGrid of the Sacramento Valley

If desired, rotate the view, apply lighting, adjust the display options, or cut cross sections through the UGrid.

# Conclusion

This concludes the “Stratigraphy Modeling – Horizons with Rasters” tutorial. Here are some of the key concepts in this tutorial:

- Rasters can be used to create UGrids.

- To use rasters with the horizons method, it is necessary to use the 3D UGrid from Rasters tool.