GMS 10.9

GMS 10.9 Tutorial

UGrid Creation

Creating unstructured grids (UGrids) in GMS

Objectives

This tutorial gives an overview about unstructured grids, or UGrids, and several methods for creating them in GMS.

Time

15–30 minutes

Required Components

GMS Core

Prerequisite Tutorials

Getting Started

1 Introduction 2 2 Getting Started 2 3 Importing a UGrid 2 3.1 Importing a 3D UGrid 2 3.2 Cell Faces 3 3.3 Importing a 2D UGrid 4 4 Creating a New UGrid 5 4.1 3D UGrid 5 4.2 2D UGrid 6 4.3 Using a Grid Frame 7 4.4 Moving and Resizing the Grid Frame 7 5 Converting from Other Object Types 9 5.1 Converting from a 3D Grid 9 5.2 Converting from a Shapefile 9 5.3 Extruding a 2D UGrid to a 3D UGrid 10 6 Using the Map to UGrid Dialog 11 6.1 Regular (Not Refined) 13 6.2 Quadtree / Octree 14 7 Horizons to UGrid 18 7.1 Viewing the 3D Quadtree UGrid 20 8 Conclusion 21

# Introduction

The UGrid module in GMS makes use of “unstructured grid” geometric objects,  referred to as “UGrids”. Similar to the 3D grids and meshes in GMS, UGrids offer greater flexibility by supporting cells with any number of faces and nodes. This allows for more realistic modeling of complex geologic features such as pinchouts. UGrids were introduced to GMS to support MODFLOW–USG, which uses the finite volume method.

There are several ways to create UGrids in GMS. This tutorial will discuss and demonstrate importing UGrids and using the New UGrid commands to create UGrids. UGrids will be created using multiple methods, including by converting other data types to UGrids, using the Map → UGrid command, and using the Horizons → UGrid command.

# Getting Started

Do the following to get started:

- If necessary, launch GMS.

- If GMS is already running, select File | New to ensure that the program settings are restored to their default state.

# Importing a UGrid

## Importing a 3D UGrid

The simplest way to load a UGrid into GMS is by importing an existing one. GMS supports the VTK unstructured grid file format.

To import a 3D UGrid from a file, do the following:

- Click Open  to bring up the Open dialog.

- Select “All Files (*.*)” from the Files of type drop-down.

- Browse to the \UGridCreation\UGridCreation directory and select “ugrid.vtu”.

- Click Open to import the file and exit the Open dialog.

- Switch to Oblique View .

A wireframe depiction of the UGrid should appear (Figure 1).

Figure 1      Imported UGrid

## Cell Faces

To review some of the UGrid display options:

- Click the Display Options  macro to bring up the Display Options dialog.

- Select Ugrid:ugrid – [Active] from the list on the left.

- Turn on Define UGrid specific options.

- Turn on Cell faces.

- Click OK to exit the Display Options dialog.

The cell faces should be visible with different colors for different layers (Figure 2). The colors come from the materials assigned to the cells.

Figure 2      UGrid with cell faces turned on

- Use the Rotate  tool to view the UGrid from different angles.

This UGrid consists of prismatic wedge elements with an irregular boundary and appears to have three layers. However, as an unstructured grid, a UGrid does not require continuous layers—or any layers at all. All cells in this grid are 3D.

To review the UGrid Properties dialog, do the following:

- In the Project Explorer, right-click on “ ugrid” and select Properties… to open the UGrid Properties dialog.

Notice the number of 2D and 3D cells; this UGrid is composed of only 3D cells.

- Click OK to close the UGrid Properties dialog.

## Importing a 2D UGrid

To review another example of an imported UGrid:

- Click the New  macro to start a new project.

- If asked to save, click Don’t Save.

- Click Open  to bring up the Open dialog.

- Select “All Files (*.*)” from the Files of type drop-down.

- Select “cow.vtu” and click Open to import the project and exit the Open dialog.

A UGrid should appear showing a wireframe cow (Figure 3). This type of unstructured grid is not very useful for groundwater modeling, but it illustrates the flexibility of the UGrid module in GMS.

Figure 3      Imported cow unstructured grid

- Use the Rotate  tool to view the UGrid from different angles.

- In the Project Explorer, right-click on the “ cow” UGrid and select Properties… to open the UGrid Properties dialog.

Note the number of 2D and 3D cells: this UGrid is a hollow shell made up of 2D cells. While not particularly useful for groundwater modeling, 2D UGrids in GMS serve as projection grids for generating 3D UGrids, as discussed later.

- Click OK to exit the UGrid Properties dialog.

# Creating a New UGrid

Another simple way to create a UGrid in GMS is by using the UGrid options in the New menu, accessible by right-clicking in the Project Explorer. These commands and their dialogs function much like those used to create new 2D and 3D structured grids.

## 3D UGrid

- Click the New  macro to start a new project.

- If asked to save, click Don’t Save.

- Right-click in either the Project Explorer or the Graphics Window and select New | UGrid 3D… to bring up the New UGrid dialog (Figure 4).

This dialog closely resembles the one used to create structured 3D grids. It allows for specifying the grid’s origin, dimensions, and the number of cells in the X, Y, and Z directions. Alternatively, cell sizes can be defined, with the option to apply bias, causing cell sizes to gradually increase or decrease across the grid. A more effective method for cell biasing will be discussed later in this tutorial.

Figure 4      New UGrid dialog

- Below the Z-Dimension section, enter “10.0” as the Rotation about Z-axis.

- Click OK to close the New UGrid dialog.

- Switch to Oblique View .

A 3D UGrid should be visible (Figure 5). Notice that the cells are all 6-sided 3D objects, or hexahedrons. This method always creates UGrids with hexahedron cells in 3D and quadrilateral cells in 2D. The creation of more complex cell types will be covered later in the tutorial.

Figure 5      New 3D UGrid

## 2D UGrid

The New | UGrid 2D… command functions similarly but creates a UGrid comprised of only 2D cells. The dialog is the same except that only the option to specify cells in the Z dimension is available. Feel free to experiment with this command.

## Using a Grid Frame

While the New UGrid dialog provides a quick and easy way to create a UGrid, it requires predefined values for the origin and lengths of the grid in each dimension. A more intuitive approach is to create a grid frame that surrounds the existing data, allowing the grid to be oriented graphically before using the New UGrid dialog.

- Click the New  macro to start a new project.

- If asked to save, click Don’t Save.

- Click Open  to bring up the Open dialog.

- Select “Project Files (*.gpr)” from the Files of type drop-down.

- Select “biscayne.gpr” and click Open to import the project and exit the Open dialog.

A conceptual model should be visible including well points and arcs representing rivers and canals.

- Right-click in either the Project Explorer or the Graphics Window and select New | Grid Frame to create a new grid frame.

Now it is necessary to fit it around the data.

- In the Project Explorer, right-click on “ Grid Frame” in the Project Explorer and select Fit to Active Coverage.

## Moving and Resizing the Grid Frame

While the Fit to Active Coverage command is convenient, it is possible to rotate the grid frame or resize it to better fit the data.

- Using the Select Grid Frame  tool, select the frame and rotate, resize, and reposition the grid frame to more or less match the image shown in Figure 6.

- Drag the corner or side anchor points of the grid frame to resize it

- Click anywhere inside the grid frame, or on an edge—but not on an anchor point—to drag and move the entire grid frame

- Click and drag on the circle in the lower right corner to rotate the grid frame

Figure 6      Grid frame surrounding a conceptual model

- When finished positioning the grid frame, right-click in the Project Explorer and select New | UGrid 3D… to bring up the New UGrid dialog.

Notice that the origins, lengths, and rotation are defaulted to values that come from the grid frame.

- Click OK to accept the defaults and close the New UGrid dialog.

GMS creates the UGrid inside the boundary of the grid frame (Figure 7). If this were a real model, more cells might need to be created.

- Turn off the “ ugrid” in the Project Explorer.

Figure 7      The UGrid created inside the grid frame

# Converting from Other Object Types

UGrids can be created from several other types of objects in GMS, including TINs; 2D grids, 3D grids, and meshes; polygons in a coverage; and polygon shapefiles in the GIS module. Two example conversions are demonstrated here.

## Converting from a 3D Grid

The current project includes a 3D Grid that is turned off. It is possible to convert it to a UGrid.

- Fully expand the “ 3D Grid Data” folder in the Project Explorer.

- Turn on “ grid” then right-click and select Convert To | UGrid.

- Select Yes to the prompt to only convert active cells used by MODFLOW.

There are inactive cells in the 3D grid that can’t be seen because they are set to zero in the MODFLOW IBOUND array. By selecting yes, GMS will not include those cells in the UGrid.

- Turn off “ grid” so the UGrid is visible.

- Click the Display Options  macro to bring up the Display Options dialog.

- Select Ugrid: ugrid (2) – [Active] from the list on the left.

- Turn on Define UGrid specific options.

- Turn on Cell faces.

- Click OK to close the Display Options dialog.

The UGrid should now look like the 3D grid it came from (Figure 8).

Figure 8      UGrid created from the 3D grid

## Converting from a Shapefile

A UGrid can be created by conversion from a shapefile.

- Turn off “ Map Data” and “ UGrid Data” in the Project Explorer.

- Turn on “ GIS Layers”.

A shapefile should appear in the form of a quadtree grid (Figure 9). Quadtree UGrids can be created in GMS or imported and then converted to UGrids as shown here.

Figure 9      Shapefile showing a quadtree grid

- Zoom  in on the shapefile to better see the quadtree refinement.

- When done examining the grid refinement, Frame  the project.

- In the Project Explorer, right-click on “ Map Data” and select New Coverage….

- Expand “ GIS Layers” then right-click on “ biscayne.shp” and select Convert To | Feature Objects to open the first page of the GIS to Feature Objects Wizard dialog.

- From the Select a coverage for mapping drop-down, select “new coverage”.

- Select Next > to go to the next step of the GIS to Feature Objects Wizard dialog.

- Accept the default settings and click Next >.

- Accept the default settings and click Finish to close the GIS to Feature Objects Wizard dialog.

- Under the “ Map Data” folder, right-click on “ new coverage” and select Map To | Polygons → UGrid.

- Turn off “ biscayne.shp”.

- Zoom  in to see that the quadtree refinement matches the shapefile.

## Extruding a 2D UGrid to a 3D UGrid

Now the UGrid consists of 2D cells. To create a MODFLOW model with the grid, a 3D UGrid is needed. To extrude the 2D UGrid into a 3D UGrid, do the following:

- Switch to Oblique View .

- Zoom  on one of the edges of the grid to see that the cells have no depth.

- Click the Toolbox  macro to open the Toolbox (beta) dialog.

Under the Tools tab, expand the “ Unstructured Grids” folder.

Double-click on the “ Extrude UGrid” tool to open the Extrude UGrid dialog.

For the Input grid, select “UGrid Data/ugrid (3)”.

Note that the Extrude UGrid tool allows defining multiple layers for the new 3D UGrid. For this example, only one layer will be generated.

For the Layer 1 thickness, enter “100.0”.

For the Output grid, enter “biscayne 3d”.

Click OK to close the Extrude UGrid tool dialog and to start the progress window for the tool

- Click OK to close the Extrude UGrid progress window.

The new UGrid is now 3D and therefore has depth (Figure 10).

Figure 10      2D UGrid extruded to a 3D UGrid

# Using the Map to UGrid Dialog

Another method for creating UGrids is through the Map → UGrid dialog. This approach offers more advanced tools compared to previous methods seen previously, allowing the creation of quadtree, octree, and Voronoi UGrids based on a conceptual model. Additionally, the UGrid boundaries can be aligned to match those of the conceptual model.

- Click the New  macro to start a new project.

- If asked to save, click Don’t Save.

- Click Open  to bring up the Open dialog.

- Select “Project Files (*.gpr)” from the Files of type drop-down.

- Select “mapToUGrid.gpr” and click Open to import the project and exit the Open dialog.

- Fully expand “ Map Data” and turn on “ Grid Frame” in the Project Explorer.

This opens a grid frame and conceptual model with two coverages (Figure 11).

Figure 11      Conceptual model and grid frame

The conceptual model defines an area of interest for building a groundwater model. In this setup, the top arc is designated as a constant head boundary, while the remaining boundary arcs are defined as no-flow boundaries. A central arc represents a stream, and two well points are included—one of which is located in a separate coverage. Additionally, a rectangular region in the lower right part of the model has been identified for increased grid refinement.

To map the feature objects to create a new UGrid and look at the options:

Select the “ Grid Frame” so it’s active.

Click the Map → UGrid  macro to bring up the Map → UGrid dialog (Figure 12).

The first drop-down is Dimension, which allows you to choose between either “2D” or “3D”.  The next option is UGrid type, which has four choices: “Regular (not refined),” “Quadtree / Octree,” “Nested Grid,” and “Voronoi”. While the nested grid option is not covered in this tutorial, the other three types are best explained through demonstration.

Figure 12      Map → UGrid dialog

## Regular (Not Refined)

For this example, the grid has 10 cells in the X and Y dimensions and three cells in the Z dimension. GMS uses the grid frame to determine the size of the grid, while the number of cells determines the size of each individual cell. Either specify the number of cells directly, as shown here, or define the target cell size and let GMS determine how many cells will fit within each dimension. When using a target cell size, keep in mind that GMS may slightly adjust the size to prevent partial cells from appearing along the grid lines.

- Click OK to accept the defaults and close the Map → UGrid dialog.

Notice the rectangular cells. This is what is meant by the “Regular (not refined)” option. Also notice that the cells outside the outer polygon boundary (the grid frame) are not included in the UGrid. They are not inactive—they simply don’t exist.

Switch to Oblique View .

The UGrid is 3D with three layers and fits the dimensions of the grid frame. It consists of 10 cells in both the X and Y dimensions (Figure 13).

Figure 13      Regular (not refined) appearance

## Quadtree / Octree

### Quadtree: All Layers the Same

To see what the quadtree option produces:

- Switch to Plan View  and turn off the new “ ugrid”.

- Click the Map → UGrid  macro to open the Map → UGrid dialog.

- From the UGrid type drop-down, select “Quadtree / Octree”.

- From the Refine points drop-down, select “Use refine points in active coverage only”.

- From both the Point refinement and Arc/poly refinement drop-downs, select “Refine all layers the same”.

- Click OK to close the Map → UGrid dialog.

Notice that the grid has been refined in a quadtree pattern around the arcs, points, and polygons (Figure 14). In a quadtree grid, each cell has, at most, two neighboring cells along each side. The largest cells match the size of those in the regular grid. This means the cell count specified in the Map → UGrid dialog for the X, Y, and Z dimensions refers to the unrefined grid—what the grid would look like without any refinement.

Turn on the Single layer option in the UGrid Viewing Mode  toolbar and change the layer to “2” then “3”.

The Single layer option allows viewing one layer of the UGrid at a time. Notice that all layers are refined the same. This tutorial will now cover how to define the refinement in the conceptual model.

Figure 14      Quadtree UGrid

### Refinement Attributes

Refinement is specified as attributes on feature objects in the conceptual model.

- Turn off the new “ ugrid (2)” in the Project Explorer.

- Select “ Map Data”.

- Using the Select Arc  tool, double-click one of the arcs along the top of the coverage to bring up the Attribute Table dialog.

Notice that the Refine attribute is on and that the Base size (ft) is set to either “100.0” or “200.0”, depending on the selected arc. UGrid cells along this arc will be about 100 (or 200) feet in size.

- Click Cancel to close the Attribute Table dialog.

- Using the Select  tool, double-click the well point in the upper right area to bring up the Attribute Table dialog.

Notice that the Refine attribute is on and the Base size (ft) is set to “75.0”. This means the UGrid cell at this location will be 75 feet in size. The Bias and Max size options are ignored when creating quadtree UGrids. These options are only used when creating structured 3D grids.

- Click Cancel to close the Attribute Table dialog.

- Using the Select  tool, double-click the rectangular polygon in the lower area to bring up the Attribute Table dialog.

Refine is on and Base size (ft) is set at “100.0”, so all cells in this area of the UGrid will be about 100 feet in size.

- Click Cancel to close the Attribute Table dialog.

- Double-click somewhere in the main polygonal area to bring up the Attribute Table dialog.

Notice that Refine is off. Cells in this area will be allowed to grow to the maximum allowed size.

Click Cancel to close the Attribute Table dialog.

Double-click on the “ M1” coverage in the Project Explorer to bring up the Coverage Setup dialog.

Notice the Refinement attribute is on in the list of Sources/Sinks/BCs. This must be on to have the Refine attribute available for points, arcs, and polygons.

- Click Cancel to close the Coverage Setup dialog.

### Quadtree/Octree

To review the Octree option for points:

- Using the Select  tool, double-click the well point in the upper right area to bring up the Attribute Table dialog.

- Select “Points” from the Feature type drop-down.

- Enter “3” in both the From layer and To layer columns.

- Click OK to close the Attribute Table dialog.

- Click the Map → UGrid  macro to bring up the Map → UGrid dialog.

- From the Point refinement drop-down, select “Octree”.

- Click OK to accept the defaults and close the Map → UGrid dialog.

- Use the UGrid Viewing Mode  toolbar to view layers 2 and 3.

Note that each layer has a different level of refinement. Layer 3—where the point is located—is the most highly refined. The “Octree” option supports both vertical and horizontal refinement. As a result, every cell can have up to two neighboring cells on either side, and up to four neighboring cells above or below. However, this option does not subdivide layers into additional sublayers. The grid still consists of just three distinct layers across the entire UGrid.

The octree option is also available for arcs and polygons and works similarly.

### Quadtree on Assigned Layers

To look at the quadtree on assigned layers option:

- Turn off the new “ ugrid (3)”.

- Click the Map → UGrid  macro to bring up the Map → UGrid dialog.

- From the Arc/poly refinement drop-down, select “Quadtree on assigned layers”.

- Click OK to close the Map → UGrid dialog.

- Use the UGrid Viewing Mode  toolbar to view layers 2 and 3.

Note that quadtree refinement around arcs and polygons occurs only in layer 1. This is because the Quadtree on assigned layers option refines cells only in the layer(s) to which arcs or polygons are assigned. In this case, all arcs and polygons are assigned to layer 1 by setting the Layer range attribute in the Coverage Setup dialog, and specifying 1 for both the From layer and To layer values for each arc or polygon. As a result, the transition from layer 2 to layer 3 is more abrupt, lacking the smoothing effect seen with the Octree option.

While this approach results in fewer cells—which can significantly reduce computation time—it comes at the cost of reduced accuracy. However, if groundwater flow is primarily horizontal, this might be an acceptable tradeoff.

For more practice creating quadtree UGrids, the “MODFLOW-USG – Quadtree” tutorial also discusses and demonstrates quadtree creation.

### Voronoi

The last option to look at in the Map → UGrid dialog is the Voronoi option.

- Turn off “ ugrid (4)” in the Project Explorer.

- Click the Map → UGrid  macro to bring up the Map → UGrid dialog.

- Select “Voronoi” from the UGrid type drop-down.

Note that most other options become unavailable when using Voronoi grids. This is because Voronoi UGrids are fundamentally different from the other types; the number of cells in the X and Y dimension cannot be specified. Instead, cells are generated using a meshing and triangulation algorithm based solely on the geometry of the points, arcs, and polygons in the active coverage.

- Click OK to accept the defaults and close the Map → UGrid dialog.

- Click anywhere in the Graphics Window.

- Switch to Plan View .

A Voronoi UGrid features a distinct type of cell. Cells are constructed so that boundary conditions are applied at the cell center, and the faces between adjacent cells are perpendicular to the line connecting those centers (Figure 15). This configuration improves computational accuracy when applying the finite volume method.

Figure 15      Voronoi UGrid on layer 1

For Voronoi grids, refinement attributes on feature objects are ignored; only the geometry of the feature objects influences the grid. To achieve finer resolution, vertices must be redistributed along arcs. In this case, increasing vertex density along the arc defining the rectangular polygon in the lower right results in smaller cells sizes in that area.

### Refining the Polygon Arc

- Turn off “ ugrid (5)” in the Project Explorer.

- Select “ M1” to make it active.

- Using the Select Arcs  tool, select the arc that forms the rectangular polygon in the lower-right area of the larger polygon.

- Right-click on the arc and select Redistribute Vertices… to open the Redistribute Vertices dialog.

- Enter “100.00” for the Average spacing.

- Click OK to close the Redistribute Vertices dialog.

- Click Map → UGrid  to bring up the Map → UGrid dialog.

- Click OK to accept the defaults and close the Map → UGrid dialog.

Now the cells in the area of the polygon are much smaller (Figure 16).

Figure 16      The area of the rectangular polygon is more refined

# Horizons to UGrid

So far, the methods used to generate UGrids have produced either 2D UGrids or 3D UGrids with continuous, flat layers in the Z dimension. Elevation data for layer boundaries can be used to interpolate to MODFLOW layer arrays, allowing the UGrid to be warped after creation. This process is the same as that used for structured 3D grids and is described in the “MODFLOW – Interpolating Layer Data” tutorial.

Another way for generating UGrids allows for the creation of layers that are not flat and may even pinch out. This method uses the Horizons → UGrid command and is covered in detail in the “MODFLOW-USG –Complex Stratigraphy” tutorial.

To briefly look at this method, do the following:

- Click New  to start a new project.

- If asked to save, click Don’t Save.

- Click Open  to bring up the Open dialog.

- Select “Project Files (*.gpr)” from the Files of type drop-down.

- Select “horizons.gpr” and click Open to import the project and exit the Open dialog.

A project containing a lot of data—including boreholes, cross sections, and TINs—will appear (Figure 17). All of this data is used in the converting from a horizons conceptual model to UGrid.

Figure 17      The horizons conceptual model

- Select “ TIN Data” in the Project Explorer to make it active.

- Select TINs | Horizons → UGrid… to bring up the Horizon Elevations page of the Horizons to UGrid dialog.

- In the Conceptual model section, turn on Use horizons conceptual model.

- Click Next > to go to the Top and Bottom Elevations page of the Horizons to UGrid dialog.

- In the Primary UGrid section, select “ voronoi”.

- In the Top elevation section, select TIN elevations.

- Click Next > to go to the Build UGrid page of the Horizons to UGrid dialog.

- In the Meshing options section, turn on Minimum element thickness and enter “2.0” in the field below that.

- Click Finish to create the 3D UGrid and close the Horizons to UGrid dialog.

## Viewing the 3D Quadtree UGrid

The 3D UGrid should be generated within a few moments, depending on the speed of the computer used. It is considered best practice to assign a more descriptive name to the new UGrid; however, for the purposes of this tutorial, it will not be renamed.

- Turn off “ Borehole Data” and “ Solid Data” in the Project Explorer.

The 3D UGrid should appear similar to Figure 18.

Figure 18      3D Voronoi UGrid made using the Horizons → UGrid command

- On the UGrid Viewing Mode  toolbar, turn on Single layer.

- Review layers 1–5 using the toolbar.

Note that this UGrid includes some layers that are discontinuous, or “pinched out”. The top and bottom elevations of each layer are irregular, conforming to the solid, borehole, and cross-section data.

Feel free to experiment with the Horizons → UGrid command. For further practice, refer to the “MODFLOW–USG – Complex Stratigraphy” tutorial.

# Conclusion

This concludes the “UGrid Creation” tutorial.  The following key concepts were discussed and demonstrated in this tutorial:

UGrids can be created in GMS in the following ways:

Importing

New UGrid commands

Conversion from other objects

Map → UGrid command

Horizons → UGrid command

The Map → UGrid command can create the following types of UGrids:

Regular (not refined)

Quadtree/Octree

Voronoi

The Horizons → UGrid command can create complex UGrids with layers that pinch out

The nested grid option creates a grid with rectangular cells that are refined like the quadtree option, but the surrounding cells are not smoothed.